# IRAF User’s Manual

for

- Sproul AP-6 CCD Image Data

compiled by:

- H.J. Augensen

M.R. Schultz

- Revised:    1999 September 07

- TABLE OF CONTENTS

| Introduction | Overview of the IRAF Package ……………….……………………………….. | Page 2 |
| --- | --- | --- |
|  | Part I. Preliminary Steps |  |
| Section 1 | Login & Startup XWindows ………………..………………………………….. | Page 6 |
| Section 2 | Setting Up the Process Directory……………………………..………………... | Page 6 |
| Section 3 | Starting IRAF and Converting .fit into .imh Files ……………………………... | Page 8 |
|  | Part II. Processing the Images |  |
| Section 4 | The IRAF Reduction Package CCDRED ..……………………………………… | Page 9 |
| Section 5 | Combining Bias, Dark, and Flat Frames to Produce Single Master Frames ….. | Page 9 |
| Section 6 | Correcting the Object Files…………..…………………………………………. | Page 10 |
|  | Part III. Photometry to Obtain Magnitudes |  |
| Section 7 | Displaying and Examining Processed Images ……………………..……………. | Page 11 |
| Section 8 | Preliminary Photometry with IMEXAMINE ……………….….……….………... | Page 12 |
| Section 9 | Precise Photometry with APPHOT ……………..…..………..…………………... | Page 13 |
|  | Part IV. Miscellaneous Information & Ending Steps |  |
| Section 10 | Changing Directories and Checking Hard Drive Space …………….………….... | Page 15 |
| Section 11 | Exiting IRAF and Logging Out ……………………………………………….… | Page 16 |
|  | Appendices & References |  |
| Appendix A | The Apogee AP-6 CCD Chip ………………………………………………………… | Page 17 |
| Appendix B | Calibration Frames: Biases, Darks, and Flats …………………………………… | Page 17 |
| Appendix C | Table of FWHM Values for APPHOT Package …………………………………… | Page 19 |
| References | ………………………………………………………………………………….. | Page 20 |

- Introduction.    Overview of the IRAF Package*

This manual deals with the procedures involved in extracting stellar magnitudes from CCD images obtained at the Sproul Observatory and the National Undergraduate Research Observatory (NURO) using IRAF (Image Reduction and Analysis Facility) command language, the standard image processing and analysis program used by professional astronomers. IRAF was developed at the National Optical Astronomy Observatories (NOAO) in Tucson, AZ, and is available on the internet at the IRAF website:

http://iraf.noao.edu/

Images taken with the Sproul 24-inch refracting telescope are 1024 ´ 1024 FIT images produced with the PMIS operating system and an Apogee AP-6 thermoelectrically-cooled camera. The specs of the AP-6 camera can be found in Appendix A.    Images taken at NURO are 512 ´ 512 images from a Photometrics liquid-nitrogen-cooled camera. The image is stored in .FIT format, which contains both image header and the pixels comprising the image in a single large file.    Previously, we used IRAF format, which consists of two separate files, one small file containing image headers (.imh) and the other (invisible to the user) containing the pixels, but this has been discontinued.

There are two parts to the data reduction. The first is processing the raw images. The second is measuring the processed images to obtain magnitudes.    Each is discussed below in some detail.

1.    Processing the Raw Images

During the course of the evening, one takes images of star fields containing variable stars or other target objects of interest. These "object" files need to be processed to remove spurious patterns introduced in the CCD camera by electronic readout noise, thermal electrons, and pixel-to-pixel sensitivity variations. To remove these effects, one takes a large number of calibration images during the course of the evening. There are three types of calibration images. (1) Zero (or "bias") frames: Typically 20 to 40 of these are taken each evening to establish the pattern of readout noise across the chip. They are taken with zero exposure time; the chip is just flushed and read out. (2) Dark frames: Typically 20 to 40 images are taken each evening, with 120 second integration times, but with the shutter closed. This establishes the pattern and rate at which thermal electrons accumulate in the CCD chip. Since dark current is negligible with the nitrogen-cooled camera at NURO, it is not absolutely necessary to take dark frames there, but they must always be taken at Sproul. (3) "Flat Field" images: usually 5 or 10 images through each filter (B,V,R, and/or I) are taken while pointing at blank sky near sunset or sunrise. (A uniformly illuminated screen is sometimes used.) The purpose is to determine the pixel-to-pixel pattern caused by sensitivity variations of the pixels or shadowing or "vignetting" by telescope optics or dust particles.    A more detailed explanation of these three types of calibration frames is provided in Appendix B.

Each object file must be corrected for these effects. To do this an IRAF command called    zerocombine is used to create a "master" Zero frame by averaging together all the individual zero frames. Similarly, a program called darkcombine is used to create a master Dark frame by averaging together all the individual dark frames — the program automatically subtracts the master Zero frame from each raw dark frame before

averaging. Next a program called flatcombine is used to create master Flat frames for each filter, again by averaging together the individual flat-field images, after first subtracting the master Zero frame and the Dark frame (scaled to the exposure time of the Flat). At NURO the images are also trimmed to a smaller size than that created by the camera, and use the trimmed region, called the "overscan" to remove large-scale patterns from the readout background. Once this is set at the startup of IRAF, however, the "trim" and "overscan" functions are automatic and should not be of concern under ordinary circumstances.

When master Zero, Dark and Flats have been generated, a program called ccdproc is run.    Ccdproc automatically subtracts the Zero and Dark (scaled to exposure time) from each object file, and divides by the appropriate Flat for the filter used.    Once ccdproc has been run, the object files, now zero-, dark-, and flat-corrected, are ready to be measured.

2.    Measuring The Processed Images To Extract Magnitudes

There are several ways to extract magnitudes from a CCD image. The images of stars on the processed

frames, in profile, look like a Gaussian or bell-shaped curve. The magnitudes are extracted from the images by measuring the area under the curve, which represents light from both the star and the sky background, subtracting out the sky background, dividing by the exposure time, and taking the log of the result. The final product is a number that represents the brightness of the star---it's a magnitude. (Technically speaking it's an "instrumental magnitude", since it's not referred to a standard star in the sky, but just calculated from the counts you get in your CCD.)

Two IRAF commands commonly used to extract magnitudes are imexamine and apphot. Both commands essentially let you point the cursor to a star, push a mouse button, and print out an instrumental magnitude. For quick-look, moderate precision measurements one can use imexamine, but in order to ensure the best results, one should use apphot, which allows the user to set the measuring parameters interactively, insuring that the proper sky background is chosen, that adjacent stars don't contribute to the measured brightness, that a sufficient amount of the star image is sampled, etc. The use of imexamine and

apphot is beautifully discussed in a write-up by Bentley Laaksonen.

STARTING IRAF

IRAF is started differently on different machines, but usually by simply typing "cl". Once started, you should see the "command language" prompt.

cl>

One enters commands just as if using any Unix or DOS machine. A list of useful IRAF commands is listed in the IRAF document by L. Marschall listed in the References.    IRAF commands always have associated with them a "parameter" file, which sets the operating parameters for the file. You need to learn how to set these parameters for your particular needs, using the epar (edit parameters) command. Some hints for what parameters to set for good results, but you may find your own particular recipe that works better.

Initially, when you start IRAF, you should run the command

setinstrument

which will set up some of the fundamental parameters you need for your observing site. The computer responds to this command by requesting the instrument ID info, to which you respond:

Instrument ID (type ? for a list):    	ap6 	(for Sproul)

tek512 	(for NURO)

To accept the default, just press return. You will find yourself immediately in the editor for several of the parameter files needed for data reduction. The first is the parameter file for the CCD reduction system, ccdred. Usually you don't have to edit this. To accept the choices, just push the <Esc> key, type a colon (:), and then type

:q (or wq). 	This is the standard procedure for exiting the editor.

You will find yourself next in the parameter editor for the program ccdproc, which is the master program that trims your images to size, applies zero, dark, and flat field corrections if needed, etc. Make sure all the parameters are set to the proper values for your site and your purpose.    When you have edited all the ccdproc commands that need setting, you can then type Esc :q to get back to the cl> prompt. You are ready to go.

STEPS IN PROCESSING IMAGES

1)    Creating a List of .FIT Files for Batch Processing the Images

If you're converting lots of files, which is usually the case, IRAF lets you take lists from a data file. You can create a file which contains the FITS file names. This is called fit.list.    When editing the input parameter in IRAF, you type

@fit.list

The result will be the extraction of headers and pixels from the <filename.fit> files which appear in fit.list.    Note that the "at" sign, @, is essential. That indicates that input comes from the file. Otherwise IRAF thinks that fit.list is the name of a single file to be converted instead of a listing of files to be converted.

2) Making A Master Zero Frame

First make sure you have all your "bias" or "zero" frames in IRAF format. Say they're named bias01.fit, bias02.fit, etc. Then make sure the parameters for the zerocombine command are set (See Section 4), using the command epar zerocombine. When the parameters are set properly, exit the editor using Esc :wq and then simply type zerocombine.    Even simpler, after setting the parameters and before exiting the editor, type :go, and this will begin the zerocombine process.    The computer now reads all the zero frames, averages them, and outputs the results in the file Zero.fits, which is now the master Zero image.

If you want to make sure the image came out OK, type imstat *.fits to check the statistics of each image. You may also want to display the image on the screen to make sure the that Zero.fits looks smoother than any of the individual bias??.fits files.

3) Making a Master Dark Frame

The procedure is exactly the same as zerocombine.    Edit the darkcombine parameters (see Section 4), and then just type :go.    The computer reads all the dark frames, subtracts the master zero frame created in the last step, and averages them together into a master dark frame,. Dark.fits.

4) Making Master Flats in Each Filter

Again, the procedure is the same as zerocombine and darkcombine. Edit the flatcombine parameters (see Section 4). When done, type :go. IRAF then reads each flat field image, subtracts Zero and Dark from it, averages together ones of the same filter, and prduces a master flat for each filter. A single flatcombine command produces a set of master flats, e.g. FlatB.fits, FlatV.fits, FlatR.fits, and FlatI.fits.

Note that the IRAF program reads the headers to determine the filters, and will only combine flats for the same filter. Thus the name you give as an output file name is a "root" name, to which the filter name will be added when the master flat is created. For instance, when you specify a root name "Flat*", IRAF will name the output files FlatI, FlatR, FlatV, FlatI, depending on the filter. Check these images out using the display command to make sure your flats look reasonable. This is a matter of judgment, and as you get more experience in processing you'll learn what to look for.

5) Processing the Object Files

The last step is to make sure any object files you want process are in IRAF format. You can edit the ccdproc parameter file with the command epar ccdproc. After making sure the ccdproc parameters are set properly (see Section 6), simply type :go, and IRAF will completely process any images on the directory.    If you get any errors saying it can't find a flat-field, dark, or zero image, make sure that the names for these files in the ccdproc parameter file match the actual master files you have on your directory. You can display the images to check if the pattern noise has been removed and the background looks relatively smooth. Next, you are now ready to measure magnitudes using imexamine or apphot.

6) Compressing the Processed .FIT Files for Storage or Transfer (optional)

You may want to compress the images before storing or ftping over the internet. The command gzip will do this, creating a file with an added extension .gz, which is the extension for unix compressed files. So file image6.fits becomes image6.fits.gz when it is compressed. This saves a fair amount of space on processed images.    Note, this doesn't do much with raw images that don't have a lot of pixels of similar value. There are other compression algorithms (such as compress) that may used also.

*Adapted primarily from the document, CCD Photometry Using IRAF at Gettysburg College and the National Undergraduate Research Observatory, by Laurence A. Marschall, available on the Web at: www.gettysburg.edu/academics/physics/clea/irafrduc.html

Part I.    Preliminary Steps

Section 1.    Login & Startup XWindows

1.1    Login & Starting Up Xwindows

On physics PC, start linux (if not already in it) and log in with username & password.

To initiate xwindows, type:      startx

1.2    Opening xterm, xgterm, & ximtool Windows

Now open three windows in xwindows, as follows:

- Click on xterm button on the vertical panel at right. This opens an xterm window. Upper-left square in the Desktop on the lower right panel is highlighted.    Adjust size & location of window.

- Now click on the Desktop square just to the right of this.    Then, on the vertical panel click first on the xgterm button,    and then click on the ximtool button. (Alternatively, you may open ximtool in a third window separate from xgterm.)

- Adjust the three windows just opened to a convenient size & location on the screen.

Section 2.    Setting Up the Process Directory

2.1    Changing to Sproul Directory

In xgterm (middle) window, first change to the sproul directory.    This is where all the night directories containing the raw .FIT files are kept:

cd      /data1/sproul

To see what night directories are contained in the sproul directory, type at prompt:    ls

2.2    Changing to A Process Directory Within Sproul Directory

Each process directory now has the name proc followed by the extension of the night on which the files were taken, so the general form is proc.yymmdd.    For instance,    proc.990611 is the process directory for all files taken on the night of 1999 June 11.      To change to this process directory from sproul directory, type:

cd      proc.990611

Then type at prompt:    ls    to display the files which proc.990611 contains.    Note that some night directories have the extension .zdf, which indicates that zero, dark, and flat frames were taken that night.

2.3    Copying .FIT Files from <observing date> Directory to Process Directory

In xgterm (middle) window, assuming you are in a particular process directory (e.g., proc.990611), copy .FIT files you are going to work on from <observing date> directory to the process directory.    For example:

cp    ../990611/*.FIT.gz . 		if files have .gz extension

cp    ../990611/*.FIT .  		if files do not have .gz extension

To check that the copy has gone well, type:    ls    to display the files.

Now, delete any BIAS, DARK, or FLAT files using remove (rm) command.    Note: if you have already started IRAF (> cl prompt displayed), use delete command (del) in place of remove:

rm BIAS*

rm DARK*

rm FLAT*

The reason for the deletion is that some of these files may be bad, and so they will be copied from a master directory which holds a set of bias, dark, and flat frames which are known to be usable.

2.4    Copying Zero, Dark & Flats from ZDF.<observing date> Directory to Process Directory

The files containing bias (zero), dark, and flat frames are now stored in a directory with the prefix ZDF and the extension the date of observation, with general form ZDF.yymmdd.    For instance, ZDF.990528 contains only the (unprocessed) calibration frames which were taken on the night of 1999 May 28, and perhaps some additional calibration frames taken on nights close to this.

Look at the ZDF.yymmdd directories to locate a set of calibration images that were taken at an earlier date that is closest in time to the date of observation of your current images. While in your process directory, copy the calibration images from this directory to the process directory:

cp    ../ZDF.990528/*.FIT.gz    .

Note that if calibration (bias, dark, or flat) images were taken on the same date as the star images you copied in Section 2.3, then they must be deleted, in case one or more of the original files is defective.

2.5    Unzipping Files    (*needed for all files which have .gz extension)

To unzip the .FIT.gz files in the process directory, type at the    > prompt:

gunzip *.FIT.gz		if you have not yet started IRAF ( > prompt)

!gunzip *.FIT.gz    	if you have already started IRAF (cl> prompt)

2.6    The mkiraf Command    (*should already have been done in each process directory)

The command mkiraf must be executed in each process directory.    It sets up the necessary subdirectories and prepares an environment for running IRAF.    It should already have been executed in each process directory.    If, for some reason, it has not yet been run (e.g., a new process directory), then change to the directory and type mkiraf, and respond to terminal type: xgterm

Part II.    Processing the Images

Section 3.    Starting IRAF & Converting .fit to .imh Files

3.1    Starting IRAF

To start IRAF (if not already started, as indicated by cl> prompt),    type (at prompt):

>    cl

3.2.    Converting .fit Files to .imh Files

At cl> prompt, type:

cl> files *.FIT    >    fit.list      		creates file called  fit.list

View (and if necessary edit) list by first typing:

cl> vi fit.list

Look at the list.    If it looks ok, proceed to the next line below.    If there are any unwanted entries to be removed (usually not), do so by moving the cursor to the line with the file name and hitting d twice (dd).    Then, save these changes by typing      :w

Next change all the .fit endings to .imh endings using:

:g/.FIT/s/.FIT/.imh

Now save the new list as image.list with:

:w image.list

Now exit vi editor:

:q				quits vi editor

Then, edit the rfits parameter list:

cl> epar rfits			edit rfits parameters

fits_fil = @fit.list

iraf_fil = @image.list

:go			begins process of creating .imh files

Lastly, delete all *.FIT files, which are no longer needed:

cl> del *.FIT      			deletes all *.FIT files

*Note:    From this point on the instructions will use all lower case for file names, use upper case where appropriate. Files transferred from the observatory by FTP will usually be all upper case.

Section 4.    The IRAF Reduction Package CCDRED

4.1    Starting CCDRED

cl>    noao

no> imred

im> ccdred

4.2    Setinstrument for AP6 Camera

cc> setinstrument site=sproul

Respond to Instrument ID: ap6    <Enter>

:q

This brings up a second window.    Type in the following (if they are not already present):

images = @image.list

zero = Zero

dark = Dark

flat = Flat*

:wq			quits and saves parameters

Section 5.    Combining Bias, Dark, and Flat Files to produce Single Master Frames

5.1    Combining Bias (Zero Exposure) Frames

cc> ccdlist *.imh	    ccdtype=zero	lists CCD processing info on all bias files

cc> epar zerocombine		edit the zerocombine parameter list

input = @image.list

process = yes

:go			combines all bias files into a single file Zero.imh

5.2    Combining Dark Frames

cc> ccdlist *.imh    ccdtype=dark	lists CCD processing info on all dark files

cc> epar darkcombine		edit the darkcombine parameter list

input = @image.list

process = yes

scale = none

:go			combines all dark files into a single file Dark.imh

5.3    Combining Flat Frames

cc> ccdlist *.imh    ccdtype=flat	lists all .imh files which are flats

cc> epar flatcombine		edit the flatcombine parameter list

input = @image.list

output = Flat

process = yes

subsets = yes

:go			creates combined Flat.imh file for each filter.

5.4    Deleting Original BIAS, DARK, & FLAT Frames

Now that the master frames Zero.imh, Dark.imh, and Flat.imh have been created, you can delete individual bias (zero), dark, and flat frames, since they are no longer needed.

cc> imdel BIAS*

cc> imdel DARK*

cc> imdel FLAT*

*Note:    If bias frames have base name different from BIAS, then use that base name in imdel command above.    The same applies for dark and flat frames.

Section 6.    Correcting the Object Files

6.1    Correcting All Object Files with Master Bias, Dark, and Flat Frames

cc> epar ccdproc

images = @image.list

:go				divides each file by combined flat

*Note: This may take several minutes, depending on the number of files to be processed.    Take a break!

When done, the cc> prompt returns.    To verify that corrections have been made for all object files, type:

cc> ccdlist *.imh ccdtype=object		corrected files should have displayed [TZDF]

6.2    Exiting the ccdred, imred, and noao Packages

Once finished, exit the iraf image correction packages by typing in xgterm window:

cc> bye

im> bye

no> bye

You should now be back to the root IRAF screen with cl> prompt.    You may now (1) change to a new process directory and process more files.    In this case, type:    cd    ../proc.yymmdd, where yymmdd is the date of the new process directory, then begin again at Section 2.3; (2) proceed to Part III, Section 7 and do photometry on these images; or (3) proceed to Part IV, Section 11 in order to quit this session.

Part III.    Photometry to Obtain Magnitudes

Section 7.    Displaying and Examining Processed Images

7.1    Starting IRAF (if already started, go to next subsection, Running Apphot)

If you are continuing from the previous sections with IRAF already up and running, go on to the next step.

Be sure to be in process directory with IRAF running and cl> prompt displayed.    If not, type (at prompt):

>    cl

7.2    Running Apphot

The photometry package apphot will allow us to determine magnitudes.    To access it, type:

cl>    noao

no> digi

di>    apphot

The prompt displayed should now be ap>.

7.3    Editing Display Settings    (*very important!)

Edit the display settings for optimal viewing.    Type:

ap> epar display

fill = yes		*Note: if fill = no, the displayed star field will be unrecognizable!

:wq

7.4    Displaying Image Files

Next, to see what files are contained in the directory, type at prompt:

ap> ls

As many as four images can be displayed in the ximtool buffer.    Usually, we display the R frame in buffer 1 and the V frame in buffer 2.    If the comparison star is on separate frames from the target star, then display the comp R frame in buffer 3 and the comp V frame in buffer 4.

ap> disp    filename    1		displays given image in buffer 1 in ximtool window

ap> disp    filename    2		displays given image in buffer 2 in ximtool window

etc. 			etc.

7.5    Orienting Frames

Each frame displayed in the ximtool window should be oriented so that North is up and East to the left, as is viewed on the CCD computer. In most cases, the frame will be inverted up-down from this orientation, and so follow the procedure outlined:

- Click on vertical up-down arrow above panner in upper right corner of ximtool window.    This will invert image vertically to achieve desired effect.

- Repeat this in each of the four buffers.

- Once set, you do not need to do this again in this buffer unless you close and later reopen the window.

Section 8.    Preliminary Photometry with Imexamine

8.1    Starting Imexamine

We wish to obtain statistics of the target star and each comparison star image in the CCD frame, before proceeding to the precise photometry technique. First, in xgterm with window active, type

ap> imexamine		gets statistics on the image needed in datapars below

Note:    After typing imexamine, the xgterm window will no longer respond to commands.

8.2    Measuring FWHM, Sky, Peak Threshold, and Preliminary Magnitudes

After typing imexamine in xgterm window, perform the following steps:

- Click on ximtool button at bottom of page: this will take you to ximtool window, where the image will be displayed with blinking circular cursor (annulus).

- The ximtool window should be active (bar displayed at top is blue). If not (bar is gray), hit <Enter> or click on image.

- Move cursor to target star and type “a”.    This prints out stats on image in xgterm window. Check this now in xgterm window, then return to ximtool window and repeat for each star of interest in field.

- Alternatively, with cursor on star, type “r”, which produces a radial profile of the star image. Record Mag, Peak Threshold, Sky, Ellipticity and FWHM values (the 10th, 8th, 7th,    5th and 2nd numbers from the right, respectively).    Also note that the 7th value from the right is useful in determining the minimum good sky value (subtracting about 50 to 150 counts from this number is usually a good value to use in the apphot package.

- After completing measurements on all star images in the ximtool buffers, you will need to exit imexamine in order to load (display) more images in the four buffers.    To quit imexamine mode, type “q” anywhere within the image displayed in active ximtool window (be sure top bar is blue).

- Repeat these steps for each image file in the directory.    From the measured FWHM, you will need to calculate 1.7 FWHM and 2.5 FWHM, to be used in the magnitude calculations below. These values can be readily calculated using the tables included in Appendix C.    After making these measurements for all stars in the directory, proceed to the steps below, where you will set the parameters to be used in calculating fluxes from star and sky, and subsequently instrumental star magnitudes.

8.3    Exiting Imexamine

To exit imexamine mode, move to imexamine window, make active (blue bar at top), then type “q” on active image in ximtool window.

Section 9.    Precise Photometry with APPHOT

9.1    Setting Parameters for Apphot

ap> datapars

scale = 1				you should not have to change this

fwhmpf = FWHM in pixels  	FWHM found by examining image above

sigma = 20			default for AP6 CCD chip

datamin = sky value 		sky value found from examining image above

datamax = 50000			default for AP6    CCD chip

readnoi = 19.0			default for AP6    CCD chip

epadu = 3.5			default for AP6 CCD chip

exposu = EXPTIME		keyword in header for exposure time

airmass = AIRMASS 		keyword in header for air mass

filter = FILTER			keyword in header for filter (B,V,R)

obstime = TIME-OBS		keyword in header for time of observation

itime = INDEF

:wq

ap> findpars

thresho = 1.7 FWHM in pixels	    FWHM = 2.35s,    so 1.7 FWHM    »    4s

:wq

ap> fitskypars

annulus =    2.5 FWHM in pixels	inner diameter of annulus in pixels

dannulus = 10-15			width of annulus band, between 10 and 15 pixels

skyvalue = minimum good value 	usually ³    100 counts (above bias level)

For long (5-10min.) exposures this can be

as high as 1000.

:wq

ap> photpars

aperture = 1.7 FWHM in pixels

:wq

ap> epar daofind

image =    filename.imh 		e.g., image = NSV1166R.imh

:go

You will now be prompted to verify the values of the parameters entered.    Press press <Enter> after each prompt to accept the default value, unless you want to change something:

FWHM of features:    <Enter>		accepts default value already entered

standard dev of background: <Enter>	accepts default value of 20

detection threshhold in s:	 <Enter>	accepts default value already entered

minimum good data value: <Enter>	accepts default    value used in fitskypars routine

maximum good data value: <Enter>	accepts default value of 50000

ap> epar phot

image =    filename.imh		e.g., image = XXCYGR.imh

coords = default

interact = no

verbose = yes

:go

You will now be prompted to verify the values of the parameters entered.    Unless you want to change something, just press <Enter> after each prompt to accept the default value.

centering algorithm = 	<Enter>

centering box = 		<Enter>

etc. …		<Enter>

min good value = 	<Enter>

max value = 		<Enter>

A list of star images with various parameters including magnitudes will appear. Write down the instrumental magnitudes of the two or three stars that you are interested in, identified by their coordinates.

To view this list again using the vi editor, type at the prompt:

> vi <filename.coo.*>

> vi <filename.mag.*>

9.2    Using txdump Command to Print Out a List of Magnitudes

To print out a list of magnitudes for each object frame using the apertures and sky annuli specified above,    type:

> txdump    filename.imh.mag.1    image,id,mag,merr,flux,sky,ifilter

9.3    Using tvmark to Display Numbers on the Image    (Optional)

To display numbers on graph, type:

> epar tvmark

label = yes

number = no

:go

9.4    Exiting the apphot, digiphot, and noao Packages

When finished with photometry, exit iraf image correction packages by typing in xgterm window:

ap> bye

di>    bye

no> bye

You should now be back to the root IRAF screen with cl> prompt.    Proceed to Part IV, Section 10.1 if you wish to change to another process directory and process more files, or to Section 11 in order to quit this session completely.

Part IV.    Miscellaneous Information & Ending Steps

Section 10.    Changing Directories and Checking Hard Drive Space

10.1    Changing To Another Process Directory

After completing the processing and/or photometry of the files in a single night, do not delete any files.    Instead, change to the next process directory you want to work on and repeat the steps above as necessary.    For instance, if you are currently in the directory proc.990611 and want to change to proc.990619, type:

cd    ../proc.990619

- If you are going to continue to do photometry on star images in this new process directory (this assumes the images have already been corrected for zero, dark, and flat calibration),    then go back to Part II, Section 7.3 (if you are still in the IRAF photometry package apphot) or to Section 7.2 (if you have already exited the photometry packages and are back at the cl> prompt) and proceed from there.

- If you are going to process raw .FIT files in this new directory, first exit the three IRAF photometry packages with a triple bye (see Section 9.4).    The cl> prompt should now be showing.    Now go back to Part I, Section 2.3 and proceed from there.

- If you wish to quit IRAF and end your session, proceed to Part IV, Section 11.

10.2    Checking Directory Path

Occasionally you may forget what directory or subdirectory you are in.    To check this type at prompt:

pwd

You will then see displayed the path (e.g., /data1/sproul/proc.990611)

10.3    Checking Hard Drive Space

Since the processed images occupy a large amount of space on the system hard drive (over 2 Mb per image), it is a good idea to periodically check on the amount of disk space used.    To do this type the command:

df    -k

This lists the amount of space currently used as a percent of space available in each partition of the hard drive.    The most important one is data1. If this exceeds 90%, contact Marty Schultz or Prof. Augensen.

Section 11.    Exiting IRAF and Logging Out

When ready to go home, you should first exit IRAF , close all windows, and log out of the system using the following commands:

11.1    Exiting xgterm & xterm windows

- In the xgterm window, exit IRAF by typing at prompt: logout

- Then to close the xgterm window, simply type: exit

- Next, go to the xterm window and type:    logout

- You do not have to exit from the ximtool window which will close down on exit, so just leave it

11.2    Exiting the xwindows program

- To exit the xwindows environment, first click on the Start button at the lower left of the screen, which will display a list of options, then click on “Yes, Really Quit”.

11.3    Logging out of physics computer

- This will then display a black screen with the prompt physics>

- Type logout at the prompt, which will leave the computer in LINUX mode.

- Leave the computer on; a screensaver will kick in after about 30 minutes

Appendices

Appendix A.    The Apogee AP-6 CCD Chip

CCD array:    	1024 ´ 1024 pixels

Pixel size:    	24.6 ´ 24.6 mm

Field of view:    	8¢ ´ 8¢ at focal plane of 24-inch

Plate scale:    	0.45¢¢ / pixel at focal plane

Images: 		16 bit, each takes up 2 Mb

Gain: 		3.5

Read noise: 	19

Appendix B.    Calibration Frames: Biases, Darks, and Flats

There are three important types of calibration frames which are used to correct the star fields for noise and unevenness in sensitivity of the individual pixels. These are (1) bias (zero-exposure) frames, (2) dark frames, and (3) flat frames.    The bias and dark frames can be taken at any time during the night after the camera has cooled to its operating temperature.    Flat frames must be taken around sunset or sunrise.    Each type of calibration frame is discussed below in detail.

Bias Frames

The electronic equipment which is used to operate the telescope and CCD is one source of background noise on the images.    It can be caused by interference in the transfer of data through cables, by the motors which drive the telescope, and by any other electronics in the system.    In order to determine the electronic noise, one takes a set of "zero-exposure" frames called bias frames.    In general, the counts over a bias frame will be only a few counts above zero.    Bias frames represent the most fundamental noise in the system, and therefore they are usually the first calibration frames taken after the camera has cooled to its operating temperature.

Once taken and stored, the IRAF software reduction package combines all the bias files (from a particular night) into a single "master" bias frame. During processing, this master bias frame is then subtracted from each object file, as well as the dark and flat calibration frames,.

Dark Frames

Dark current is generated by thermal noise in the system. Dark current is due to electrons that are freed from the CCD chip because they have enough thermal energy to escape their orbits.    These thermal electrons fall into the quantum wells of the of the chip and mimic photon hits.    The higher the operating temperature, the more electrons liberated and the larger the dark current. In order to measure and correct for the dark current, one must in principle take an exposure at the same exposure time as the frame containing star images, but with the camera shutter closed. However, in practice, one takes several dark frames at different exposures, the longest being equal to the longest target star exposure.    Dark frames can be taken at any time during the night, so long as the camera is at its operating temperature.

During processing, the IRAF software reduction package combines all the dark frames (from a particular night) into a single "master" dark frame, corrected for bias noise.    This master dark frame, scaled to match to the exposure time recorded in the image header of each object file to be corrected, is then subtracted from each object frame during processing.

Flat Frames

The sensitivity of the individual pixels across the CCD array varies somewhat, often considerably. This sensitivity fluctuation is different in different filters.    Additionally, dust on the filters or other optical parts interferes unevenly with the light path. To correct for the pixel and other variations across the surface of the chip, we simply expose the chip to a uniform source of light through each of the filters that will be used that night.    The resulting flat field is usually revealed to be far from uniform, and contains an extremely uneven distribution of light areas, dark areas, dark and bright vertical lines, and donuts. The last are actually interference fringes formed by the dust particles, paint chips, or other foreign objects which have settled on the filters or filter window. If a perfectly circular dark globule appears in the center of a flat frame, it means that the chip had not cooled sufficiently at the time flats were taken. (This is because the CCD chip cools from the outside first and then progresses inward toward the center.)    New flat frames should be retaken at the next opportunity.

As with the bias and dark frames, the IRAF software reduction package combines all the flat frames in each filter into a single "master" flat frame for that filter, corrected (during previous steps) for both bias noise and dark current.    During processing, IRAF divides each object frame by the master flat taken in the same filter.    This in effect “normalizes” the pixel array of the CCD chip, so that all pixels contribute equally.    Ideally, the only differences in brightness across the field should be true ones created by the individual images of stars situated against a backdrop of much fainter sky.

This document is available on the Internet at Prof. Augensen's home page:

- http://www.science.widener.edu/~augensen/

Appendix C.    Table of FWHM Values for APPHOT Package

|  | 1.7 x | 2.5 x |  |  | 1.7 x | 2.5 x |  |  | 1.7 x | 2.5 x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FWHM | FWHM | FWHM |  | FWHM | FWHM | FWHM |  | FWHM | FWHM | FWHM |
| 1.0 | 1.70 | 2.50 |  | 5.0 | 8.50 | 12.50 |  | 9.0 | 15.30 | 22.50 |
| 1.1 | 1.87 | 2.75 |  | 5.1 | 8.67 | 12.75 |  | 9.1 | 15.47 | 22.75 |
| 1.2 | 2.04 | 3.00 |  | 5.2 | 8.84 | 13.00 |  | 9.2 | 15.64 | 23.00 |
| 1.3 | 2.21 | 3.25 |  | 5.3 | 9.01 | 13.25 |  | 9.3 | 15.81 | 23.25 |
| 1.4 | 2.38 | 3.50 |  | 5.4 | 9.18 | 13.50 |  | 9.4 | 15.98 | 23.50 |
| 1.5 | 2.55 | 3.75 |  | 5.5 | 9.35 | 13.75 |  | 9.5 | 16.15 | 23.75 |
| 1.6 | 2.72 | 4.00 |  | 5.6 | 9.52 | 14.00 |  | 9.6 | 16.32 | 24.00 |
| 1.7 | 2.89 | 4.25 |  | 5.7 | 9.69 | 14.25 |  | 9.7 | 16.49 | 24.25 |
| 1.8 | 3.06 | 4.50 |  | 5.8 | 9.86 | 14.50 |  | 9.8 | 16.66 | 24.50 |
| 1.9 | 3.23 | 4.75 |  | 5.9 | 10.03 | 14.75 |  | 9.9 | 16.83 | 24.75 |
| 2.0 | 3.40 | 5.00 |  | 6.0 | 10.20 | 15.00 |  | 10.0 | 17.00 | 25.00 |
| 2.1 | 3.57 | 5.25 |  | 6.1 | 10.37 | 15.25 |  | 10.1 | 17.17 | 25.25 |
| 2.2 | 3.74 | 5.50 |  | 6.2 | 10.54 | 15.50 |  | 10.2 | 17.34 | 25.50 |
| 2.3 | 3.91 | 5.75 |  | 6.3 | 10.71 | 15.75 |  | 10.3 | 17.51 | 25.75 |
| 2.4 | 4.08 | 6.00 |  | 6.4 | 10.88 | 16.00 |  | 10.4 | 17.68 | 26.00 |
| 2.5 | 4.25 | 6.25 |  | 6.5 | 11.05 | 16.25 |  | 10.5 | 17.85 | 26.25 |
| 2.6 | 4.42 | 6.50 |  | 6.6 | 11.22 | 16.50 |  | 10.6 | 18.02 | 26.50 |
| 2.7 | 4.59 | 6.75 |  | 6.7 | 11.39 | 16.75 |  | 10.7 | 18.19 | 26.75 |
| 2.8 | 4.76 | 7.00 |  | 6.8 | 11.56 | 17.00 |  | 10.8 | 18.36 | 27.00 |
| 2.9 | 4.93 | 7.25 |  | 6.9 | 11.73 | 17.25 |  | 10.9 | 18.53 | 27.25 |
| 3.0 | 5.10 | 7.50 |  | 7.0 | 11.90 | 17.50 |  | 11.0 | 18.70 | 27.50 |
| 3.1 | 5.27 | 7.75 |  | 7.1 | 12.07 | 17.75 |  | 11.1 | 18.87 | 27.75 |
| 3.2 | 5.44 | 8.00 |  | 7.2 | 12.24 | 18.00 |  | 11.2 | 19.04 | 28.00 |
| 3.3 | 5.61 | 8.25 |  | 7.3 | 12.41 | 18.25 |  | 11.3 | 19.21 | 28.25 |
| 3.4 | 5.78 | 8.50 |  | 7.4 | 12.58 | 18.50 |  | 11.4 | 19.38 | 28.50 |
| 3.5 | 5.95 | 8.75 |  | 7.5 | 12.75 | 18.75 |  | 11.5 | 19.55 | 28.75 |
| 3.6 | 6.12 | 9.00 |  | 7.6 | 12.92 | 19.00 |  | 11.6 | 19.72 | 29.00 |
| 3.7 | 6.29 | 9.25 |  | 7.7 | 13.09 | 19.25 |  | 11.7 | 19.89 | 29.25 |
| 3.8 | 6.46 | 9.50 |  | 7.8 | 13.26 | 19.50 |  | 11.8 | 20.06 | 29.50 |
| 3.9 | 6.63 | 9.75 |  | 7.9 | 13.43 | 19.75 |  | 11.9 | 20.23 | 29.75 |
| 4.0 | 6.80 | 10.00 |  | 8.0 | 13.60 | 20.00 |  | 12.0 | 20.40 | 30.00 |
| 4.1 | 6.97 | 10.25 |  | 8.1 | 13.77 | 20.25 |  | 12.1 | 20.57 | 30.25 |
| 4.2 | 7.14 | 10.50 |  | 8.2 | 13.94 | 20.50 |  | 12.2 | 20.74 | 30.50 |
| 4.3 | 7.31 | 10.75 |  | 8.3 | 14.11 | 20.75 |  | 12.3 | 20.91 | 30.75 |
| 4.4 | 7.48 | 11.00 |  | 8.4 | 14.28 | 21.00 |  | 12.4 | 21.08 | 31.00 |
| 4.5 | 7.65 | 11.25 |  | 8.5 | 14.45 | 21.25 |  | 12.5 | 21.25 | 31.25 |
| 4.6 | 7.82 | 11.50 |  | 8.6 | 14.62 | 21.50 |  | 12.6 | 21.42 | 31.50 |
| 4.7 | 7.99 | 11.75 |  | 8.7 | 14.79 | 21.75 |  | 12.7 | 21.59 | 31.75 |
| 4.8 | 8.16 | 12.00 |  | 8.8 | 14.96 | 22.00 |  | 12.8 | 21.76 | 32.00 |
| 4.9 | 8.33 | 12.25 |  | 8.9 | 15.13 | 22.25 |  | 12.9 | 21.93 | 32.25 |
| 5.0 | 8.50 | 12.50 |  | 9.0 | 15.30 | 22.50 |  | 13.0 | 22.10 | 32.50 |

References

The following documents may be helpful in working with IRAF.    Most are available over the internet from the World Wide Web site:      http://iraf.noao.edu/

- Barnes, Jeannette:    A Beginner’s Guide to Using IRAF Version 2.10, August 1993

- Massey, Philip:    A User’s Guide to CCD Reductions with IRAF, June 1992

- Massey, Philip, and Davis, Lindsey E.:    A User’s Guide to Stellar CCD Photometry with IRAF, April 1992

- Valdes, Francisco:    User’s Guide to the CCDRED Package, revised February 1988

- Wells, Lisa A.:    Photometry Using IRAF, February 1994

The following document by Larry Marshall is a superb overview of image processing using IRAF.    Part of it is used as the foundation of the Introduction to the current document. It also provides settings for the IRAF packages ccdproc, zerocombine, darkcombine, flatcombine, and wfits.. It is available on the World Wide Web at:    http://www.gettysburg.edu/academics/physics/clea/irafrduc.html

- Laurence A. Marschall: CCD Photometry Using IRAF at Gettysburg College and the National Undergraduate Research Observatory

The following set of documents were developed at Gettysburg College by Bentley Laaksonen and Laurence Marshall.    They are an excellent introduction to the practical applications of IRAF.

- Laaksonen, Bentley, and Marschall, Laurence:    CCD Imaging, 1994

- Laaksonen, Bentley, and Marschall, Laurence:    CCD Photometry with IRAF, 1994

- Laaksonen, Bentley, and Marschall, Laurence: Photometry of Variable Stars, 1994

- Laaksonen, Bentley, and Romanishin, Bill:    Supernova Photometry, 1994