Part I.    Preliminary Steps

Section 1.    Login & Startup XWindows

1.1    Login to Physics Computer

The physics PC in Kirk 344 is in linux mode. (If not, restart computer.)    Type at physics1 login prompt:

physics1 login:    	<username>

password:    	<password>

1.2    Start Up Xwindows & Open xterm, xgterm, & ximtool Windows

To initiate xwindows, type:      startx

Now open three windows in xwindows, as follows:

- Click on xterm button on the vertical panel at right. This opens an xterm window. Upper-left square in the Desktop on the lower right panel is highlighted.    Adjust size & location of window.

- Now click on the Desktop square just to the right of this.    Then, on the vertical panel click first on the xgterm button,    and then click on the ximtool button. (Alternatively, you may open ximtool in a third window separate from xgterm.)

- Adjust the three windows just opened to a convenient size & location on the screen.

Section 2.    Setting Up the Process Directory

2.1    Changing to Sproul Directory

In xgterm (middle) window, first change to the sproul directory.    This is where all the night directories containing the raw .fit files are kept:

cd      /data1/sproul

To see what night directories are contained in the sproul directory, type at prompt:    ls

2.2    Changing to a Process Directory Within Sproul Directory

Each process directory now has the name proc followed by the extension of the night on which the files were taken, so the general form is proc.yymmdd.    For instance,    proc.990611 is the process directory for all files taken on the night of 1999 June 11.    To change to this process directory from sproul directory, type:

cd      proc.990611

Then type at prompt:    ls    to display the files which proc.990611 contains.    Note that some night directories have the extension .zdf, which indicates that zero, dark, and flat frames were taken that night.

2.3    Copying .fit Files from <observing date> Directory to Process Directory

In xgterm (middle) window, assuming you are in a particular process directory (e.g., proc.990611), copy .fit files you are going to work on from <observing date> directory to the process directory.    For example:

cp    ../990611/*.fit.gz    . 		if files have .gz extension

cp    ../990611/*.fit    .  		if files do not have .gz extension

To check that the copy has gone well, type:    ls    to display the files.

2.4    Copying Zeros, Darks & Flats from zdf.<observing date> Directory to Process Directory

The files containing bias (zero), dark, and flat frames are stored in a directory with the prefix "zdf" followed by the extension the date of observation. The general form is zdf.yymmdd.    For instance, zdf.990829 should contain only the unprocessed calibration frames which were taken on the night of 1999 August 29 and perhaps some additional calibration frames taken on nights close to this.    However, the directory may also contain the already combined master zero, dark, and flat files produced from this set, with the common extension “.fits”:    Zero.fits, Dark.fits, Flatr.fits, and Flatv.fits.

First, delete any bias, dark, or flat files from the current process directory using remove (rm) command. The reason is that some of these files may be bad;    they will be copied from a master directory which holds a set of bias, dark, and flat frames which are known to be usable.    Note: If you have already started IRAF (> cl prompt displayed), use delete command (del) in place of remove:

rm bias*

rm dark*

rm flat*

Next, find the zdf.yymmdd directory which matches the date of observations of the data in your process directory (assuming calibration frames were taken on that night) or, if calibration frames were not taken on that date, choose the next earliest zdf date.    While in your process directory, copy the calibration images to the process directory:

cp    ../zdf.990528/*.fit*    .

*If the zdf directory contains the master .fits files, then you will not need the various bias, dark, flatr, and flatv .fit files you just copied, and you will not need to do Section 5.

2.5    Unzipping Files    (*needed for all files which have .gz extension)

To unzip the .fit.gz files in the process directory, type at the    > prompt:

gunzip *.fit.gz		if you have not yet started IRAF ( > prompt)

!gunzip *.fit.gz    	 	if you have already started IRAF (cl> prompt)

*Note:    At this time files transferred from the observatory by WS-FTP appear in upper case.     The extension .fit must be converted to lower case in order for IRAF to operate successfully.

Section 3.    Starting IRAF & Making a List of .fit Files

3.1    The mkiraf Command    (*should already have been done in each process directory)

The command mkiraf sets up the necessary subdirectories and prepares an environment for running IRAF, and must be executed in each process directory.    This should already have been executed in each process directory, so there is no need to do it again.    If, for some reason, it has not yet been run (e.g., a new process directory), then change to the directory and type mkiraf, and respond to terminal type: xgterm

3.2    Starting IRAF

To start IRAF (if not already started, as indicated by cl> prompt),    type (at prompt):

>    cl

3.3    Making a List of .fit Files

At cl> prompt, type the following commands:

cl> reset imtype=fits

cl> files *.fit    >    fit.list

cl> hedit    @fit.list    exptime    "(exptime/1.0e3)"    ver-    show+

The first command above ensures that the .fit files will be used as image files. (Previously the .fit files had to be converted to .imh files before processing.)    The second command creates a list of all .fit files to be processed and puts it into a file called fit.list.    The last command is necessary to correct the .fit header of each file for an error in the exposure time recorded by PMIS.

Part II.    Processing the Images

Section 4.    The IRAF Reduction Package CCDRED

4.1    Starting CCDRED

cl>    noao

no> imred

im> ccdred

4.2    Setinstrument for AP6 Camera

cc> setinstrument site=sproul

Respond to Instrument ID: sproul    <Enter>

:q

This brings up a second window.    Type in the following (if they are not already present):

images = @fit.list

zero = Zero

dark = Dark

flat = Flat*

:wq			quits and saves parameters

Section 5.    Combining Bias, Dark, and Flat Files to produce Single Master Frames

*Note: If you are using previously created master Bias, Dark and Flat frames, skip to Section 6

5.1    Combining Bias (Zero Exposure) Frames

cc> epar zerocombine		edit the zerocombine parameter list

input = @fit.list

process = yes

:go			combines all bias files into a single file Zero.fits

5.2    Combining Dark Frames

cc> epar darkcombine		edit the darkcombine parameter list

input = @fit.list

process = yes

scale = none

:go			combines all dark files into a single file Dark.fits

5.3    Combining Flat Frames

cc> epar flatcombine		edit the flatcombine parameter list

input = @fit.list

output = Flat

process = yes

subsets = yes

:go			creates combined Flat.fits file for each filter.

5.4    Deleting the Original Bias, Dark, & Flat Frames

Now that the master frames Zero.fits, Dark.fits, and Flat.fits have been created, you can delete individual bias (zero), dark, and flat frames, since they are no longer needed.

cc> imdel bias*

cc> imdel dark*

cc> imdel flat*

*Note:    If bias frames have base name different from bias, then use that base name in imdel command above.    The same applies for dark and flat frames.

Section 6.    Correcting the Object Files

6.1    Correcting All Object Files with Master Bias, Dark, and Flat Frames

cc> epar ccdproc

images = @fit.list

:go

*Note: This command initiates the process which trims and corrects all the object files using the master bias, dark, and flat calibration frames. This may take several minutes, depending on the number of files to be processed.    Take a break!

When done, the cc> prompt returns.    To verify that corrections have been made for all object files, type:

cc> ccdlist *.fit ccdtype=object		corrected files should have displayed [TZDF]

6.2    Exiting the ccdred, imred, and noao Packages

Once finished, exit the iraf image correction packages by typing in xgterm window:

cc>    bye

im> bye

no> bye

You should now be back to the root IRAF screen with cl> prompt.    You may now (1) change to a new process directory and process more files.    In this case, type:    cd    ../proc.yymmdd, where yymmdd is the date of the new process directory, then begin again at Section 2.3; (2) proceed to Part III, Section 7 and do photometry on these images; or (3) proceed to Part IV, Section 11 in order to quit this session.

Part III.    Photometry to Obtain Magnitudes

Section 7.    Displaying and Examining Processed Images

7.1    Starting IRAF    (*if already started, go to Subsection 7.2 Running Apphot)

If you are continuing from the previous sections with IRAF already up and running, go on to the next step.

Be sure to be in process directory with IRAF running and cl> prompt displayed.    If not, type (at prompt):

>    cl

7.2    Running Apphot

The photometry package apphot will allow us to determine magnitudes.    To access it, type:

cl>    noao

no> digi

di>    apphot

The prompt displayed should now be ap>.

7.3    Editing Display Settings    (*VERY IMPORTANT!)

Edit the display settings for optimal viewing.    Type:

ap> epar display

fill = yes		*Note: if fill = no, the displayed star field will be unrecognizable!

:wq

7.4    Displaying Image Files

Next, to see what files are contained in the directory, type at prompt:

ap> ls

As many as four images can be displayed in the ximtool buffer.    Usually, we display the R frame in buffer 1 and the V frame in buffer 2.    If the comparison star is on separate frames from the target star, then display the comp R frame in buffer 3 and the comp V frame in buffer 4.

ap> disp    filename    1		displays given image in buffer 1 in ximtool window

ap> disp    filename    2		displays given image in buffer 2 in ximtool window

etc. 			etc.

7.5    Orienting Frames

Each frame displayed in the ximtool window should be oriented so that North is up and East to the left, as is viewed on the CCD computer. In most cases, the frame will be inverted up-down from this orientation, and so follow the procedure outlined:

- Click on vertical up-down arrow above panner in upper right corner of ximtool window.    This will invert image vertically to achieve desired effect.

- Repeat this in each of the four buffers.

- Once set, you do not need to do this again in this buffer unless you close and later reopen the window.

Section 8.    Preliminary Photometry with Imexamine

8.1    Starting Imexamine

We wish to obtain statistics of the target star and each comparison star image in the CCD frame, before proceeding to the precise photometry technique. First, in xgterm with window active, type

ap> imexamine		gets statistics on the image needed in datapars below

Note:    After typing imexamine, the xgterm window will no longer respond to commands.

8.2    Measuring FWHM, Sky, Peak Threshold, and Preliminary Magnitudes

After typing imexamine in xgterm window, perform the following steps:

- Click on ximtool button at bottom of page: this will take you to ximtool window, where the image will be displayed with blinking circular cursor (annulus).

- The ximtool window should be active (bar displayed at top is blue). If not (bar is gray), hit <Enter> or click on image.

- Move cursor to target star and type “a”.    This prints out stats on image in xgterm window. Check this now in xgterm window, then return to ximtool window and repeat for each star of interest in field.

- Alternatively, with cursor on star, type “r”, which produces a radial profile of the star image. Record Mag, Peak Threshold, Sky, Ellipticity and FWHM values (the 10th, 8th, 7th,    5th and 2nd numbers from the right, respectively).    Also note that the 7th value from the right is useful in determining the minimum good sky value (subtracting about 50 to 150 counts from this number is usually a good value to use in the apphot package.

- After completing measurements on all star images in the ximtool buffers, you will need to exit imexamine in order to load (display) more images in the four buffers.    To quit imexamine mode, type “q” anywhere within the image displayed in active ximtool window (be sure top bar is blue).

- Repeat these steps for each image file in the directory.    From the measured FWHM, you will need to calculate 1.7 FWHM and 2.5 FWHM, to be used in the magnitude calculations below. These values can be readily calculated using the tables included in Appendix C.    After making these measurements for all stars in the directory, proceed to the steps below, where you will set the parameters to be used in calculating fluxes from star and sky, and subsequently instrumental star magnitudes.

8.3    Exiting Imexamine

To exit imexamine mode, move to imexamine window, make active (blue bar at top), then type “q” on active image in ximtool window.

Section 9.    Precise Photometry with APPHOT

9.1    Setting Parameters for Apphot

ap> datapars

scale = 1				you should not have to change this

fwhmpf = FWHM in pixels  	FWHM found by imexamine in Sect. 8

sigma = 20			default for AP6 CCD chip

datamin = sky count 		sky count found by imexamine in Sect. 8

datamax = 50000			default for AP6    CCD chip

readnoi = 19.0			default for AP6    CCD chip

epadu = 3.5			default for AP6 CCD chip

exposu = EXPTIME		keyword in header for exposure time

airmass = AIRMASS 		keyword in header for air mass

filter = FILTER			keyword in header for filter (B,V,R)

obstime = TIME-OBS		keyword in header for time of observation

itime = INDEF

:wq

ap> findpars

thresho = 1.7 FWHM in pixels	    FWHM = 2.35s,    so 1.7 FWHM    »    4s

:wq

ap> fitskypars

annulus =    2.5 FWHM in pixels	inner diameter of annulus in pixels

dannulus = 10			width of annulus band, between 10 and 15 pixels

skyvalue = sky count + 100 	take ³    100 counts above sky background count

(for 5-10 min    exposures this can be as high as 1000)

:wq

ap> photpars

aperture = 1.7 FWHM in pixels

:wq

ap> epar daofind

image =    filename.fit 		e.g., image = NSV1166R.fit

:go

You will now be prompted to verify the values of the parameters entered.    Press press <Enter> after each prompt to accept the default value, unless you want to change something:

FWHM of features:    <Enter>		accepts default value already entered

standard dev of background: <Enter>	accepts default value of 20

detection threshhold in s:	 <Enter>	accepts default value already entered

minimum good data value: <Enter>	accepts default    value used in fitskypars routine

maximum good data value: <Enter>	accepts default value of 50000

ap> epar phot

image =    filename.fit		e.g., image = XXCYGR.fit

coords = default

interact = no

verbose = yes

:go

You will now be prompted to verify the values of the parameters entered.    Unless you want to change something, just press <Enter> after each prompt to accept the default value.

centering algorithm = 	<Enter>

centering box = 		<Enter>

etc. …		<Enter>

min good value = 	<Enter>

max value = 		<Enter>

A list of star images with various parameters including magnitudes will appear. Write down the instrumental magnitudes of the two or three stars that you are interested in, identified by their coordinates.

To view this list again using the vi editor, type at the prompt:

> vi <filename.coo.*>

> vi <filename.mag.*>

9.2    Using txdump Command to Print Out a List of Magnitudes

To print out a list of magnitudes for each object frame using the apertures and sky annuli specified above,    type:

> txdump    filename.fit.mag.1    image,id,mag,merr,flux,sky,ifilter

9.3    Using tvmark to Display Numbers on the Image    (Optional)

To display numbers on graph, type:

> epar tvmark

label = yes

number = no

:go

9.4    Exiting the apphot, digiphot, and noao Packages

When finished with photometry, exit iraf image correction packages by typing in xgterm window:

ap> bye

di>    bye

no> bye

You should now be back to the root IRAF screen with cl> prompt.    Proceed to Part IV, Section 10.1 if you wish to change to another process directory and process more files, or to Section 11 in order to quit this session completely.

Part IV.    Miscellaneous Information & Ending Steps

Section 10.    Changing Directories and Checking Hard Drive Space

10.1    Changing To Another Process Directory

After completing the processing and/or photometry of the files in a single night, do not delete any files.    Instead, change to the next process directory you want to work on and repeat the steps above as necessary.    For instance, if you are currently in the directory proc.990611 and want to change to proc.990619, type:

cd    ../proc.990619

- If you are going to continue to do photometry on star images in this new process directory (this assumes the images have already been corrected for zero, dark, and flat calibration),    then go back to Part II, Section 7.3 (if you are still in the IRAF photometry package apphot) or to Section 7.2 (if you have already exited the photometry packages and are back at the cl> prompt) and proceed from there.

- If you are going to process raw .fit files in this new directory, first exit the three IRAF photometry packages with a triple bye (see Section 9.4).    The cl> prompt should now be showing.    Now go back to Part I, Section 2.3 and proceed from there.

- If you wish to quit IRAF and end your session, proceed to Part IV, Section 11.

10.2    Checking Directory Path

Occasionally you may forget what directory or subdirectory you are in.    To check this type at prompt:

pwd

You will then see displayed the path (e.g., /data1/sproul/proc.990611)

10.3    Checking Hard Drive Space

Since the processed images occupy a large amount of space on the system hard drive (over 2 Mb per image), it is a good idea to periodically check on the amount of disk space used.    To do this type the command:

df    -k

This lists the amount of space currently used as a percent of space available in each partition of the hard drive.    The most important one is data1. If this exceeds 90%, contact Marty Schultz or Prof. Augensen.

Section 11.    Exiting IRAF and Logging Out

When ready to go home, you should first exit IRAF , close all windows, and log out of the system using the following commands:

11.1    Exiting xgterm & xterm windows

- In the xgterm window, exit IRAF by typing at prompt: logout

- Then to close the xgterm window, simply type: exit

- Next, go to the xterm window and type:    logout

- You do not have to exit from the ximtool window which will close down on exit, so just leave it

11.2    Exiting the xwindows program

- To exit the xwindows environment, first click on the Start button at the lower left of the screen, which will display a list of options, then click on “Yes, Really Quit”.

11.3    Logging out of physics computer

- This will then display a black screen with the prompt physics>

- Type logout at the prompt, which will leave the computer in LINUX mode.

- Leave the computer on; a screensaver will kick in after about 30 minutes