- Observer’s Manual

- Operating the Sproul AP-6 CCD Camera

and    24-Inch Refractor

compiled by:

- H.J. Augensen

- W.D. Heintz

M.R. Schultz

- Revised:    1999 October 08

- Table of Contents

| Chapter 1 | The Sproul Observatory ………………………………………. | Page 2 |
| --- | --- | --- |
| Chapter 2 | Open Up Procedure At Beginning of Night …………………. | Page 4 |
| Chapter 3 | Dome and Console Controls …………………………………. | Page 6 |
| Chapter 4 | Basic Observing Procedures …………………………………. | Page 9 |
| Chapter 5 | Computer and CCD Camera Startup………………………… | Page 13 |
| Chapter 6 | Taking CCD Calibration Frames …………………………….. | Page 15 |
| Chapter 7 | Taking CCD Images of Star Fields ………………………….. | Page 19 |
| Chapter 8 | Shut Down Procedure at End of Night ………………………. | Page 22 |
| Chapter 9 | Zipping and Transferring .fit Files to IRAF Computer ……... | Page 24 |
| Chapter 10 | Troubleshooting for Computer & CCD ………………………. | Page 26 |
| Chapter 11 | Emergency Procedures ……………………………………….. | Page 27 |
| Chapter 12 | Miscellaneous Information for Observers …………………… | Page 28 |
| Appendix A | Sunset & Sunrise Times at Sproul Observatory…………….. | Page 29 |
| Appendix B | Bright Double Stars for Focusing …………………………….. | Page 30 |
| Appendix C | The 24-inch Refracting Telescope …………………………… | Page 32 |
| Appendix D | The Apogee AP-6 CCD Camera ……….…………………….. | Page 34 |

- Chapter 1

- The Sproul Observatory

- 1.1    HISTORY

The Sproul Telescope was built in the period 1908-10, and has been in operation since January 1912.    It is of a type favored at that time: a long -focus refractor.    It gives only a small field of view, but that on a large scale.    The focal length of 11 m (36 ft) has the advantage that the instrument is little affected by the now increased background light on campus and from the surrounding community.    The lenses were made by the renowned optician Brashear.    The motors driving the telescope were replaced in 1966, and a console installed for easier control of the telescope motions.

The Sproul building is the third oldest on campus.    Its North part was the president’s house.    Then the dome and (later) an office wing were added to the South.

The research concentrated on the distances of nearby stars and on the exploration of double and multiple star systems. By observing positions of stars at different seasons, a small displacement (parallax) caused by the Earth’s motion may be found, from which the star’s distance can be deduced.    This works only for nearby stars, but it lays the basis for studying more and more distant stars by comparison of their radiation distributions.    So, it is the first step on the ladder of cosmic distances which now extend to faraway galaxies.

The procedure was to photograph a star over many years, and to measure its motion at a high-precision machine.    We could then attain an accuracy which corresponds to the size of a postage stamp seen from a distance of 1000 km.    The program continued for 82 years, resulting in some 90,000 plates (probably the largest archive of its kind to have come from a single telescope), and 1500 stellar distances; this accounts for about 10% of the entries in the database on parallaxes.    The program was terminated as more precise (and less expensive) techniques superseded photography.    But we had maintained a broad coverage of objects, at least for preliminary results.    Most of the stars in the volume of space surrounding    the solar system are dwarfs, smaller and much fainter than the Sun.    Observers at larger instruments can now pick the most promising of these numerous, faint targets for further study.

Stars have the feature that the majority of them occur in groups of two or more.    By studying the orbits described in such binary and multiple stars, their masses and hence the data on their evolutionary status can be deduced.    This work is done mostly visually with a micrometer (a precision screw with a high-power eyepiece), which gives a better resolving power than photographic plates.    Such work began at the Sproul refractor in 1971.    Heintz has made (including work at telescopes elsewhere) 54,000 observations of double stars over 43 years (47,500 by micrometer, 6500 by photography).    This resulted in the study of 500 orbits (a world record), and 900 newly discovered double stars are now in the catalogs with the code HEI.    In multiple systems (and those where an unseen component is present), the orbit computation may get quite complicated.

- 1.2    CURRENT RESEARCH

In 1998, the refractor was fitted with an electronic CCD camera, which has given it a new task:    the study of certain types of variable stars.    Aging stars, inflating to giant and supergiant proportions, often develop instabilities or “pulsations” in their outer layers, causing periodic and nonperiodic changes in the stars’ brightness.    Sometimes, these variations are slow and quite complex (with different periods superposed);    they require brightness measurements over months and years, preferably by the same instrument, and so many cases have not yet been adequately explored.

The types of variable stars currently under investigation at Sproul Observatory can be classed into short-period,    long-period, and suspected variables:

- Short Period Variables

- Delta Scuti Variables

- SX Phoenicis Variables

- Eclipsing Binaries – Algols & W UMa

- Long Period & Unusual Variables

- Extragalactic Supernovae

- RV Tauri and Semiregular Variables

- Possible Identification of Flare Stars

- Suspected Variables

- nsv stars suspected of variability or know to be variable with unknown type

- Chapter 2

- Open Up Procedure At Beginning of Night

- 2.1    ARRIVAL

- Get to the observatory at least 20 minutes before sunset if you intend to do flats. The camera needs at least 15-20 minutes to cool down to its operating temperature.

- Sunset times for every 10 days of each month are tabulated in Appendix A.

- Unlock and enter observing alcove at entrance to dome.

- 2.2    COMPUTER & CCD POWER

- To turn on the computer:    Turn on the power strip on top of the computer (red light should come on).    Press the ON button on the computer (the large round button).

- Enter dome and open heavy iron door in pier to turn on the power supply mounted on reverse side of door (1 is on, 0 is off).    This supplies power to cool the CCD camera. Close door carefully after flipping switch.

- Start cooling the camera, which takes about 20 minutes.    For detailed instructions, see Chapter 5, Computer & CCD Camera Startup.

*Important:

- When the computer is turned on, power is also automatically turned on for the shutter and tiny fans of the CCD camera.    This is true even is the switch in the pier is turned off.    Therefore, the computer should not be left on for long periods if the camera is not being cooled.

- 2.3    TELESCOPE POWER

- Turn on Telescope Power by turning main switch CW on upper left side of console.

- Do not turn on tracking motor (Clock Drive) until ready to begin observing stars.

- If needed, turn on Dome Lights (see Chapter 3) with switch on lower right.

- 2.4    OPEN DOME

- First make sure telescope objective is not directly below dome slit.    Open dome slit by pulling on the left side of the rope loop.

- If observing session is interrupted and observers leave the dome temporarily (due to clouds or other reasons), dome slit must be closed and main power switch turned off.

- 2.5    DOME FAN

- During warm weather, turn on dome fan located around console on right side. This will help to equilibrate the inside and outside temperatures.    If it is warm in the dome move the telescope so the camera is in front of where the fan is blowing in.    This will help the camera cool faster.    Turn off fan after equilibrium is reached.

- Chapter 3

- Dome and Console Controls

- 3.1    LIGHTS

- Dome Lights – Rotate on/off switch on console. Rotate bright/dim if necessary.    Bright turns on both lights, dim turns on only the light on the north side of the dome.

- Console Lights – Adjust knob from bright to dim, as desired.

- Setting Circle Lights (Optional) – Adjust knob until circle can be read.

- 3.2    DOME ROTATION

Rotate Forward or Reverse using either:

- switch at console or

- switch on telescope just above CCD camera.

- At Console:

- When facing the console the dome will rotate in the direction the top of the switch is pointed to.

- On Telescope:

- Forward and reverse on the telescope move the dome in the opposite direction as the console switch.

*Warnings:

- If you start to go in the wrong direction, let dome come to a full stop before reversing. Otherwise, motor damage may result.

- When rotating dome, watch that rope for opening slit does not get caught in chairs or catwalk door.

3.3    TELESCOPE FAST MOTION (SLEW)

- General Rules:

- Turn CW two clamps for RA & Dec on lower right on console so green lights go off and red lights go on (if not burned out)

- When setting coordinates, always set RA first, then Dec.

- When setting Dec, telescope can pass pier only if hour angle is not more than 20m West

- Once RA coordinate has been set using the fast motion, rotate clamp CCW so that red light goes out and green lights come back on.    Repeat for Dec.

*Warnings:

- Always be sure to watch that telescope clears pier by safe distance when moving telescope.

- Always make sure the observing ladders are not in the path of the telescope.

- 3.4    TELESCOPE SLOW MOTION

- RA:

- Make sure switches for RA & Dec have green lights on (if not, turn CCW).

- Avoid using East slow motion in RA, since it has drift.    Therefore …

- First set RA coordinate too far East (increasing RA), then bring back West slowly.

- Dec:

- Slow motion in Dec has range of ~ 6°;    thus check occasionally that it is not near the end of range (see next bullet).

- There is a small motor attached with a belt to a long worm gear mounted on the telescope tube on the camera side.    On the gear is a square block with an arm that goes up to the place the telescope is mounted to the pier.    This block should be occasionally centered on the gear using the Dec slow motion control.

- 3.5    SIDEREAL TRACKING    (CLOCK DRIVE)

- Turn on “Clock Drive” switch in upper middle part of console by rotating to "Line".

- Always turn clock drive switch to “Line” when taking images of stars.

- Always turn clock drive switch to “Off” when taking flats.

- Clock drive can be set in either on or off when taking bias and dark frames.

- 3.6    TELESCOPE LIMITATIONS IN HOUR ANGLE (HA)

- Observations should be made near HA = 0 (i.e., when the star is on the meridian).    In general, try to keep telescope within about ± 30m of meridian.    However, for both far northern and southern stars, a larger range in HA (up to ~ 1 hour) may be permissible.

- In declinations from +36° to +72°, the pier limits the westward motion of the telescope, according to the following table:

| Decl. | +80° | +70° | +60° | +50° | +40° | +38° | +37° | +36° | +35° |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HA limit West | 72m | 57m | 32m | 24m | 21m | 24m | 30m | 40m | 74m |

- 3.7    FINDER TELESCOPE

- Finder telescope has field diameter 0.9°    (radius 26¢), with limiting magnitude 9 or 10, depending on transparency & eyesight.

- Finder charts show stars down to about magnitude 9 (which should be visible even in somewhat hazy sky), especially those East of target area.    If target star is too faint to be seen, set relative to brighter stars in the field so target star will be situated somewhere in the 8¢ x 8¢ field of CCD camera.

- Chapter 4

- Basic Observing Procedures

- 4.1    GENERAL OUTLINE

- Select files from filing cabinet for stars that are to be observed that evening.    Make sure stars have right ascensions that fall within range of sidereal times between dusk twilight and dawn twilight for that date.

- Unclamp both RA and Dec (red lights on). Set coordinates coarsely using fast motion first in RA, then in Dec, according to prescription in Section 4.2 Hints for Setting Telescope, below.    Always watch that telescope does not hit pier or ladder.

- After setting RA coarsely above, clamp RA to allow slow motion (green lights on) and then fine-tune settings on dial, according to Section 4.2.    Repeat for Dec setting.    Note that the Dec dial is a bit jumpy, and good to only about ± 5¢.    The RA, however, is quite precise when used only with the West slow motion.

- Go to finder scope, look at field, then look at map and match star patterns.    Use joy stick to center on target star.    If you cannot locate field, double-check coordinates.

- When target is centered in finder, take a test exposure of 5-10 seconds. Then check field displayed on screen with DSS star chart. If star is not centered in field, use slow motion controls to adjust its position, then take another test exposure.    Note that image on screen has North up and East to the left.

- To move the star to left on image frame, move telescope West, to move right, move telescope East.

- To move star up on image frame, move telescope South, to move star down on frame, move telescope North.

- If test exposure shows star is sufficiently close to center of field, then begin taking exposures according to exposure times listed on chart:

- take one exposure in R filter

- take one exposure in V filter

- Repeat above procedure for all stars on observing list, recording all important data in log sheet.

Each of these steps (except for the first) is explained in some detail in the following sections.

- 4.2    HINTS FOR SETTING TELESCOPE

- Fast Motion in RA:

- Switch RA to fast motion (unclamps, red light will come on).

- In order to prevent wear on motors, always start at fast motion at low speed (pointer on upper-left regulator knob above switch should be turned CCW).

- crank up to high speed by turning knob CW, then CCW down to low speed again when near desired position.

- When at desired position, turn clamp on (green light will come on).

- Fast Motion in Dec:

- Same as for RA above, with additional caveats given below.

- Watch that the telescope is not more than 20m West when passing the pier!

- At declinations north of +35°, make sure that telescope tube clears pier.

- When within about 10¢, clamp Dec.

- Fine settings in RA and Dec:

- The RA dial is quite accurate; however, while setting the Dec, the RA will tend to drift a bit East.

- To get back to correct RA at telescope or console, check finder chart, which should show stars (at least one) East of target.    Use Dec slow motion (joystick) to get them into correct up/down position when viewed through finder.    Then use slow motion West to get them into correct left/right position, as shown in chart.

- At finder or console, always use West slow motion (at finder, move joystick toward telescope).    Avoid using the East slow motion: it causes drift.

4.3    TAKING A TEST EXPOSURE TO CHECK FIELD

To check if the telescope has the correct field, take a short (5 - 10 sec) exposure and view it on “Display Max Nonlinear”.

- If star and its comparison star(s) are positioned correctly (i.e., sufficiently far away from frame edge and also away from defects in chip), then proceed to take timed exposures

- If star and comparison are not positioned correctly, then apply corrections in Dec. and/or RA as instructed below:

Correction in Dec:

- Use console switch.    Slow motion (on max speed) runs ~ 2 arc minutes in 1 sec.

Correction In RA

- To move telescope West, briefly use joystick or console switch.

- To move telescope East – the best alternative method is to estimate how much in time the image is to be shifted to the right (the camera width is 8¢ = 30s of time), and to stop the clock drive for that time.

- When done, take another short test exposure.

- 4.4    SKY CONDITIONS

Sky conditions are rated according to two different criteria:    transparency and seeing, each discussed in some detail below.

- Transparency

- measures clarity of atmosphere

Examples:

- good transparency – sky is very dark, and many faint stars can be seen

- poor transparency – sky contains haze or thin clouds

- Seeing

- measures steadiness or calmness of atmosphere

- seeing scale:

1    very poor

2    fair to poor

3    good to fair

4    very good

5    excellent

Examples:

- good seeing – images are sharp & steady

- poor seeing – images are diffuse and unsteady

4.5    EXPOSURE TIMES

Suggested exposure times in each filter are provided on chart for each star.    These are generally given for very good conditions of transparency and seeing.

- If transparency and/or seeing are less than ideal, increase exposure times by 25-50%.

- If conditions are very poor, requiring 100% or more increase in exposure times, quit.

- Do not observe star fields located near a full or nearly full moon.    This is especially true if there are thin clouds present.

- 4.6    FILTER WHEEL

The Filter Wheel window is activated by clicking on the Filter Wheel icon on the Windows 95 desktop at the beginning of the observing session.      To change to a particular filter or clear aperture, key in the number according to the following list:

0		no filter

- B filter

- V filter

- R filter

- no filter

…		…

8		close down position

-1		exits program

- 4.7    LOG SHEETS

Prior to observing, record the following basic information on the log sheet:

- Name(s) of observer(s)

- Date of observation:    e.g., 1998 Aug 19/20 (since usually the observation period spans the evening of one date and early morning of the next).

- Julian Date for 00h 00m U.T.

As you observe, record the following information in the log sheet for each night:

- object

- filename for object

- filter (B, V, R)

- beginning of the exposure time (UT)

- exposure time

- seeing

- transparency

- temperature inside dome (°F)

- CCD temperature (°C)

- comments

- Chapter 5

- Computer and CCD Camera Startup

- 5.1    THE WINDOWS 95 DESKTOP

All programs and folders you need have icons on the Windows 95 desktop.    To open or close any program on the desktop, follow these general rules:

- To open a program window, double click on an icon on the desktop which corresponds to that program.

- To close a program window, clicking on the “X” button in the upper right corner of the window.

- 5.2    OPEN "ATOM TIME" (OPTIONAL)

- This synchronizes the computer clock to universal time (UTC or GMT).

- After opening, click on "Check" and then "Adjust" if necessary.

- If the time server is down you will have to wait a minute or two for the program to respond so you can close it.

5.3    OPEN "FILTER WHEEL" WINDOW

The filter wheel is controlled by a dos program running in a window.    A numbered list of slots corresponding to each filter is displayed.    For a detailed description, refer to    Section 4.6 Filter Wheel in Chapter 4.

- To change the filter, type in the number and press enter    You should hear the motor running after you press enter.

- When you are running PMIS you have to click inside the dos window to activate it and then type in the number to change the filter.

5.4  OPEN "PMIS" WINDOW

This is the camera control software, from which you can set the camera temperature, open an image window to take exposures, and (later) save images as files.    The first thing to do is set the camera temperature, explained in the next section.

5.5    SET CAMERA TEMPERATURE

This starts the cooling process for the camera.    Do this with the following commands:

- Click on "Camera" in the Apogee window.

- Then click on "Set Temperature".

- Then click on the circle to the left of "Set Temperature".

- Type in “-15” (if it is not already displayed).    Then click "OK".

5.5    OPEN "IMAGE" WINDOW

This is used to take exposures.    Detailed explanations of how to do this are provided in      Chapters 6 and 7, so be sure to refer to them.    However, the basic steps are:

- Click on "Image" in the main PMIS window.    A pulldown list will appear.

- Select "New Image" from the list.    A new window should open.

  - OPEN “ACQUIRE” MENU IN THE "IMAGE" WINDOW

This is used to set exposure times and for taking bias, dark, flat, and object images. The procedure is explained in detail in Chapters 6 and 7, which should be referred to.

*Note:    Do not use the "Flat" menu item for flats (this will automatically subtract a dark which we don't want).    Take flats as normal exposures (see Section 6.4 Flats).

*Warning:

- Do not use the computer for anything else while the camera is taking an exposure or transferring an image to the computer.    Trying to use the network is the worst case; it usually causes a bad crash requiring a shutdown and restart of the computer.

- Chapter 6

- Taking CCD Calibration Frames

- 6.1    OVERVIEW

There are three important types of calibration frames which will later be used to correct the star fields for noise and unevenness in sensitivity of the individual pixels. These are (1) bias (zero-exposure) frames, (2) dark frames, and (3) flat frames.    The bias and dark frames can be taken at any time during the night after the camera has cooled to its operating temperature.    Flat frames must be taken around sunset or sunrise.    Each type of calibration frame is discussed below in detail.

- 6.2    BIAS (ZERO EXPOSURE) FRAMES

The electronic equipment which is used to operate the telescope and CCD is one source of background noise on the images.    It can be caused by interference in the transfer of data through cables, by the motors which drive the telescope, and by any other electronics in the system.    In order to determine the electronic noise, one takes a set of "zero-exposure" frames called bias frames.    In general, the counts over a bias frame will be only a few counts above zero.    Bias frames represent the most fundamental noise in the system, and therefore they are usually the first calibration frames taken after the camera has cooled to its operating temperature.    Once taken and stored, the bias frame must be subtracted from every image taken, including the dark and flat calibration frames to be discussed below.

The procedure for taking bias frames is as follows:

- Open an image window on the PC (if you have not already done so in Chapter 5) by clicking on "Image" at the top of the PMIS window.    A new window should appear.    Click and drag it to a convenient part of the windows desktop.

- Click on "Acquire" in top portion of the image window.    This will display a pull-down list of several selections, which include "Focus", "Dark", "Flat", and "Bias".    Select "Bias" from this list.    A new image window will pop up, and a bias frame will automatically be taken.    This window can be dragged to a more suitable location.

- After the bias frame has been transferred, save the image as a file.    At the prompt in the PMIS window, type:    bsave <Enter>.    Then type the filename:    bias1.fit <Enter>.

- Repeat this procedure until five bias frames have been taken.    Save these as bias2.fit, … bias5.fit.    Each bias image will automatically be stored under directory c:\pmis\images

Later, the IRAF software reduction package will combine all the bias files (from a particular night) into a single "master" bias frame.    This master bias frame will be subtracted from each object file during processing.

6.3    DARK FRAMES

Dark current is generated by thermal noise in the system. Dark current is due to electrons that are freed from the CCD chip because they have enough thermal energy to escape their orbits.    These thermal electrons fall into the quantum wells of the of the chip and mimic photon hits.    The higher the operating temperature, the more electrons liberated and the larger the dark current.

In order to measure and correct for the dark current, one must in principle take an exposure at the same exposure time as the frame containing star images, but with the camera shutter closed. However, in practice, one takes several dark frames at different exposures, the longest being equal to the longest target star exposure.    Dark frames can be taken at any time during the night, so long as the camera is at its operating temperature.

- The general procedure for taking dark frames is as follows.

- In the Image window already open, click on "Acquire" in top portion of the image window.    Select "Dark" from the pull-down list.

- After selecting "Dark", enter the exposure time.    Use exposure times which span most of the exposures used.    For example, if your shortest object exposure was 15 sec and the longest was 8 min, with exposures of varying lengths in between, try dark exposures at 15 sec, 30 sec, 1 min, 2 min, 4 min, and 8 min.

- As each dark frame is taken, you will need to save the image as a file.    First at the PMIS prompt, type:    dsave <Enter>

- Next, type in the filename, assigning the file a name which indicates the exposure time used.    We usually take darks at 15 sec, 30 sec, 1 min, 2 min, 4 min, and 8 min, so these would be saved as, respectively:

dark15s.fit

dark30s.fit

dark1m.fit

dark2m.fit

dark4m.fit

dark8m.fit

- Each dark frame will automatically be stored under directory c:\pmis\images

Later, the IRAF software reduction package will combine all the dark frames (from a particular night) into a single "master" dark frame, corrected for bias noise.    This master dark frame, scaled to match to the exposure time in the image header of each object file to be corrected, will be subtracted from each object frame during processing.

6.4    FLAT FRAMES

The sensitivity of the individual pixels across the CCD array can vary considerably. This sensitivity fluctuation is different in different filters.    Additionally, dust on the filters or other optical parts interferes unevenly with the light path. To correct for the pixel and other variations across the surface of the array, the chip is exposed to a uniform source of light through each of the filters that will be used that night. The resulting flat field is usually revealed to be far from uniform, and contains an extremely uneven distribution of light areas, dark areas, dark and bright vertical lines, and donuts. The last are actually interference fringes formed by the dust particles on the filters or filter window.

The procedure for taking flats is as follows:

- In order to take flat frames at sunset, arrive at the dome at least 20 minutes before sunset in order to perform startup, as explained in previous sections.    This is necessary in order to cool the camera to -15°C by sunset.

- Move the telescope to a clear patch of sky well above the horizon, preferably near the zenith.

- Make sure telescope tracking is off.    Thus, any star images which appear in the exposure will be located in different parts of the frame on each exposure. This will allow IRAF to median out the star images from a sequence of two or three images.

- Set filter wheel to V filter. This can be done at any time after the computer has been turned on.    If you wish to take B flats (normally not), do them first, before V.

- A few minutes before sunset, after the camera has cooled to its operating temperature, begin taking flats at 1.0 sec exposures by using the normal exposure procedure for stars (Acquire, Expose,    1 sec), not the one labeled “Flats” under the Acquire menu.    Do not save the images yet unless the count rate has reached a suitably low value (see next step).

- Check count rate as each image is downloaded.    If exposures are begun shortly before sunset, count rate will be positive several thousand counts for first few frames.    The ideal count for flats is around 0 (i.e., halfway between -32,000 and +32,000). When counts fall below +10,000, begin to save the images using "flsave" (as opposed to "osave" for the object (star) images).    Type:    flsave <Enter>.    A dialog box will pop up and prompt for the filter and filename. Be sure to enter lower case for the filter used: v, not V.    Take a total of three (or at least two) to five exposures in the V filter and name them:

flatv1.fit    	flatv2.fit	flatv3.fit	etc.

- Repeat this procedure for the R filter, again entering lower case r, not R, when prompted for the filter.    Save them as:

flatr1.fit 	flatr2.fit 	flatr3.fit	etc.

As with the bias and dark frames, the IRAF software reduction package will combine all the flat frames in each filter into a single "master" flat frame for that filter, corrected for both bias noise and dark current.    During processing, IRAF divides each object frame by the master flat taken in the same filter.    This in effect “normalizes” the pixel array of the CCD chip, so that all pixels contribute equally.    Thus, when the light from a field of stars falls on the pixel array, located in the focal plane of the telescope, the only differences in brightness across the field should be true ones created by the individual images of stars situated against a backdrop of much fainter sky.

*Important Notes:

- Never use the “Flat” feature under the “Acquire” menu.    Flats are exposed in the same way as ordinary star fields (although they are saved using flsave instead of osave).    This is in contrast to the procedure for taking bias and darks frames, where the exposures are taken with the "Bias" and "Dark" .

- If when taking flat exposures the count rate drops below around -10,000, increase exposure time to 1.5 or 2.0 s. Do not use exposure times larger than around 10 s for flats, as this will often result in star images appearing in the frame.    If the count rate has fallen below -20,000, then the sky has gotten too dark to get flats in that filter.

- The CCD chip cools from the outside first and then progresses inward toward the center.    If a perfectly circular globule appears in the center of a flat frame, it means that the chip has not cooled sufficiently by the time flats are being taken.    The circle will become progressively smaller and eventually disappear completely as the cooling approaches its set point.

- If irregular dark blobs appear in the field, this means that dust, paint chips, or other foreign objects have settled onto the camera window. From time to time, you will need to take the camera off and remove these objects from the camera window.    New flat frames should be retaken at the next opportunity.

- Chapter 7

- Taking CCD Images of Star Fields

7.1    FRAME ORIENTATION

The CCD frame represents an 8' x 8' square segment of the sky.    North is up and East is to left, as would be the case in direct observation of the sky at a point on the meridian.

- 7.2    DETERMINING THE PROPER TELESCOPE FOCUS

The focus of the telescope is temperature sensitive, and therefore as temperature fluctuates from night to night, and even during the night, so does the location of the focal plane of the telescope. Therefore, it is a good policy to determine the best focus for each night.    This is most efficiently done early in the evening while the CCD chip has not yet cooled to its final temperature (around -15° C), and while the sky is still not dark enough to begin taking target star images.    This can be done just after taking flat frames (if applicable) shortly after sunset.    The procedure is as follows:

- Appendix B lists close double stars to be used for focusing.    This list is also kept in a folder in the observing alcove.    Choose a double star with RA close to the current sidereal time (read from the console). Set the telescope on the chosen double.

- Take a set of 1 - 2 sec frames while varying the focus to determine the best focus for the telescope. The telescope focus is read from the metal ring just above the camera, and can be adjusted by grasping one of several handles projecting from the ring. Use increments of about 20 - 40 units in changing the setting.    To increase, rotate wheel slowly to right.    To decrease, rotate wheel to left and slightly below the desired setting, then rotate right.    Focus should lie somewhere in range 450-550.    Note: in general, as temperature increases, the focus setting decreases.

- Once optimal focus has been achieved, record the setting value and also the temperature off the thermometer in the dome.

7.3    TARGET STAR IMAGES

- Take a 10 sec exposure to check that desired target (or comparison) star is centered in field.    If star is very faint (mag = 12 -16) or transparency is poor, use 20 sec.

- Use the "Display" menu and set the "Scaling" to "Optimal Non Linear" to see faint objects on an exposure.    This is useful for centering objects using short exposures.

- If star is not centered, use the slow motion controls in RA and Dec on the console or at the telescope to move the star into a more suitable position in the frame.

- If single field contains both target and comparison stars, adjust RA and Dec so that the point halfway between the two objects is close to the center of the field.

- Note that there is a column of "hot" pixels running vertically to the left of center frame. If your target star falls on one of these, there should be no problem, as the flat field will compensate for it.

- However, there is a partial column or two of "cold" pixels in the lower right of the frame. These should be avoided, if possible.

- 7.4    DETERMINING PROPER EXPOSURE TIMES

In PMIS, the count range varies between around -32,000 counts to + 32,000 total counts, for a total range of about 64,000 counts.      Counts should ideally be around half saturation, which would read around zero.    Adjust exposure times accordingly, but try not to exceed half saturation. On the other hand, 20% saturation (around -20,000 counts) is adequate, and may be desirable since by increasing the exposure time tracking error is introduced.

7.5    SAVING IMAGES AS FITS FILES

There are four macros to save the images as FITS files.    Simply type in the name of the macro in the main PMIS window after acquiring an image.    Something must be typed in for each dialogue box that is displayed when the macro runs.    If you leave one blank the macro will quit and you will have to run the macro again.    These macros will automatically add the appropriate FITS headers for IRAF to use when processing the images.    The four macros, the first three of which have been already been discussed in Chapter 6, are summarized below:

- bsave is used to save bias (zero) frames.

- flsave is used to save flat frames.

- dsave is used to save dark frames.

- osave is used to save object frames.

To save an object frame after exposing a star field, type: osave <Enter>.    You will then be prompted to enter information on the following, always in lower case:

- initials of observer(s),

- filter used, r, v, or b,

- airmass, usually taken as 1,

- object name, and

- filename

It is best to use a filename which encodes the name of the star, the filter used, and whether or not it is a comparison field.    For example, suppose the frame contains the RV Tauri variable star CE Virginis imaged with the red filter.    We would probably name it:

cevirr.fit

If the field contains the comparison star (only) for CE Vir, taken in the R filter, then use:

cevirrc.fit

*Important:    Use lower case r, v, b when asked for filter type in any macro.

All images are saved to c:\pmis\images.    The folder called "new data" on the desktop is a link to this directory.

- Chapter 8

- Shut Down Procedure at End of Night

- 8.1    EXIT PMIS

- Close the main dialog box and answer “Yes” to the “Save Program State” box.

- Before closing the filter wheel program put the filter wheel in position number 8, wait for it to stop and then exit the program by entering “-1” for the filter number and press <ENTER>.

- 8.2    MOVE FILES FROM NEW DATA TO DATA ARCHIVE

At the end of the night, image files must be transferred from "New Data" to "Data Archive".

- Open the data archive folder and make a new folder using the current date as the folder name.    Use the format yymmdd (May 28,1999 would be 990528).    If bias, dark, and/or flat frames were taken on this date, add the extension .zdf (or .f if only flats were taken, .zd, if only bias and darks, and so on.    Open this new folder.

- Next, open the “New Data” folder and select all the files inside.    Then drag the files into the new folder you just made using the right mouse button and select “move” when you let go of the button.

- Now close all the open folders on the desktop.    There are other ways to do this; if you are familiar with Windows 95 you may move the files using your favorite method.

- Later, you can transfer the .fit files in the night directory to the IRAF computer for processing.    See Chapter 9.

  - CLOSE DOWN WINDOWS 95

- Once the files have been moved shut down Windows 95 using “Start”, “Shutdown”, “Yes”.    When Windows 95 shuts down the computer will turn off.

- Turn off the power strip (red switch) on top of the computer table.    Do not shut off the power strip until the computer has shut down.

8.4    TURN OFF CCD CAMERA

- Open door at base of pier and turn off CCD power supply. Off is “0” position.    You may do this any time after exiting PMIS.

8.5    CLOSE DOWN DOME/TELESCOPE

In general, reverse steps for start up:

- Move telescope to rest position aligned with polar axis:    Set Dec to near +90° first (make sure telescope will clear the pier) then move East to near 18h.    Use the RA setting circle on the telescope so you can see if the telescope is going to run into anything.

- Rotate dome slit to face southwest (the opposite side from the entrance).

- Close dome by pulling on rope (the right side of the loop). Give one last tug at end to ensure tight seal.

- Turn off clock drive and main power on console.

- Close dome door, turn off light in observing alcove, and lock alcove door behind you.

- 8.6    LAST CHECK BEFORE LEAVING

- Dome slit closed?

- Main power on console turned off?

- CCD power switch in pier door turned off?

- Computer turned off?

- Door to observing alcove locked?

- Chapter 9

- Zipping and Transferring .fit Files to IRAF Computer

At some point, the .fit files which were acquired during the night must be compressed (zipped) and then transferred from the CCD computer to the IRAF computer where the processing will take place to obtain magnitudes. Ideally, this should be done at the end of the night, when the file transfer rate across the internet is fairly fast, but usually the observers are too tired after long hours of observing.    Therefore the procedure may be postponed until the following day (when the file transfer rate will be slower).    But first we must make sure that all the .fit files to be transferred are in lower case.

- 9.1    CHANGING FILE NAMES TO LOWER CASE

- On desktop window, click on "Data Archive" icon and then right-click on the night directory which you want to want to transfer (e.g., 990809).

- Note that the first letters of all. fit files stored under PMIS are capitalized.    If left unchanged, the transferred files will appear on the IRAF computer with all their letters capitalized. The files must be in lower case before they are processed.

- Carefully left-click on the first .fit file in the directory three times. The first click highlights the file, and the second click initiates "rename" mode.    The third click should be done with the vertical cursor placed at the beginning of the filename.    Delete the first capital letter ("Num Lock" key must be off), then type in the same letter in lower case.    Press <Enter> to save.    Repeat for the remaining files.

- 9.2    ZIPPING .FIT FILES

- On desktop window, click on "WinZip" icon, then on "I Agree" button.

- If you have not done so already, click on "Data Archive" icon and then right-click on night directory which you want to want to zip (e.g., 990809).    Hold down right mouse button and drag entire directory into the WinZip window, then release.

- A Drag & Drop box will appear.    Click on "New" button.

- Where it asks for "file name", type in the name of the night directory, then click OK.

- Then click on "Add" in previous box.    Compression should now begin — look at bottom panel for status.

- Afterwards, the compressed (zipped) files should now appear in a subdirectory of format <directory name>.zip (e.g., 990809.zip) under the original directory.

- 9.3    TRANSFERRING FILES TO IRAF COMPUTER

- Double-click on the WS_FTP icon on the MS windows desktop, then click on "OK" button.    Type in password, then click OK.

- Two windows will appear: a small "Session Profile" window positioned in front of the main WS_FTP window behind it.

- In the smaller "Session Profile" window in the foreground, you may be prompted for a password.    If so, enter it and left-click on "OK" in the upper right corner. Otherwise, just left-click on the "OK".    This will cause this window to disappear.    Wait a few seconds while the connection between the CCD computer and the IRAF computer (This is currently located in the physics office at Widener University).

- The main WS_FTP window should now dominate the screen.    The left side represents the home (CCD) computer, the right side the remote (IRAF) computer.    Double-click on the local directory on the left side, which will bring up a list of subdirectories.

- On the left (home side), select (by left-clicking on) the .zip directory containing the files you want to transmit to the remote computer.    If you have more than one directory to transmit, select all of them by holding down the <Ctrl> key while left-clicking on each directory.

- Click with the left mouse button on the right-arrow key ® to initiate the transfer process.    The transfer of all the files in the directory may take several minutes, depending on the number of directories to transmit, on the total number of files in each directory, and on the time of day or night when the transmission occurs.

- When transfer is complete, all the directories transferred should appear in the right remote window.

- Exit the WS_FTP program by clicking on the "Exit" button in the lower right corner of the screen.

- Chapter 10

- Troubleshooting for Computer & CCD

10.1    COMPUTER PROBLEMS

If you get an interface error with a “-1” displayed:

- Try closing PMIS and running it again.    If you still get the error when downloading you will have to shutdown the computer and turn it back on (restarting windows will not fix this).    This will mean you have to set the camera temperature again when you run PMIS and wait for it to stabilize (usually 10 to 15 minutes).

- If images become very noisy when displayed in pmis:

- Closing and restarting PMIS will not usually fix this. You will have to restart windows by clicking on: “start”, “shutdown”, the “restart computer” circle, and then “OK”.    This will restart windows and when you run PMIS the images should look normal again.    Since the power is not turned off the camera temperature is unaffected by this.

If the computer locks up:

- Try <CTRL>-<ALT>-<DEL>.    If a dialog box pops up and has any program listed as “not responding”, click on it and then click on the “end task” button.    If windows is still locked up use <CTRL>-<ALT>-<DEL> again and click on the “shutdown” button.    This should close windows and turn off the computer.    You will have to turn it back on and set the camera temperature again.    If <CTRL>-<ALT>-<DEL> does nothing, then you will have to shut off the computer and turn it back on.

10.2    CCD AND FILTER WHEEL PROBLEMS

If CCD flat field has circular globule in exact center:

- The CCD chip has not uniformly cooled to a stable temperature – the center is still warm while the outer edges have cooled.    Start cooling earlier next time!

If CCD flat field contains dark irregular blobs:

- These represent dust or paint chips that have accumulated onto the CCD window. Detach the camera and remove the dust or paint chips with a camels hair brush or lens paper attached to the eraser end of a pencil.

If CCD temperature is not stable:

- Check the temperature in the dome, the camera can only cool to 35 °C below the ambient temperature.    You can try to run at a higher temperature (-10°C), but you will have to take new dark frames for processing the images.

- If the filter wheel makes a much louder noise than usual:

- When this happens it is safest to close the filter wheel program and restart it.    This will cause the filter wheel to find position number 1 again.    Sometimes the filter wheel program will close by itself when this happens.

- Chapter 11

- Emergency Procedures

- 11.1    DOME & TELESCOPE CONTROLS

- In case telescope function fails:

- Check fuse lights (right side of console).    The light comes on when a fuse is blown.

- Spare fuses are located in box in shelves on wall to right of console.

- In case a power failure occurred before the observing night:

- Standard time and sidereal time dials need to be reset manually by sliding out the glass and moving the hands.

- RA dial needs to be reset by centering on a star and using the RA adjust switches.

- If above equipment or other problems cannot be remedied:

- call Prof. Heintz during evening shift (before midnight): 543-4359

- leave a note on observing desk detailing the problem during morning shift

11.2    PERSONAL SAFETY

In case of suspicious persons outside the observatory or an intruder in the building:

- call Campus Security immediately:    8333

- In case of fire:

- call Campus Security immediately:    8333

- As a backup for all emergencies

- Swarthmore Police Telephone number is 543-0123

- General emergencies:    Dial 911

- Chapter 12

- Miscellaneous Information for Observers

- 12.1    SPROUL TELEPHONE NUMBERS

- Heintz office:    				610 - 328 - 8265

- Observer's Desk (rings softly)		610 - 328 - 8270

12.2    LOCAL WEATHER REPORT

- Philadelphia Region Weather Telephone:    215-936-1212

- Also, check bookmarked Weather Web Sites on CCD Computer

- 12.3    LOCAL TIME SIGNALS

- Telephone Number:    215-846-1212

12.4    VISITORS

- Visitors are nor permitted in the dome or elsewhere in Sproul Observatory during nighttime hours.    This is for insurance purposes and for safety, because building is kept dark during observing sessions.

- If you wish to bring friends in or use the 6-inch telescope, get authorization from Professors Heintz or Gaustad

- Keys to the Sproul Observatory should never be loaned to a friend or family member unless that person is to take your place on an observing night.    Permission from Professor Heintz must first be obtained.

- 12.5    MUSIC

- It is not advised to play music during observing session, but if you do, be sure volume is kept low.    Be aware that students may be studying a short distance away.

12.6    BREAKS

- If you become tired and need to rest for extended period (more than 30 minutes), be sure to turn off telescope drive and close dome

- Keep out of Sproul offices of faculty & staff

- Do not make private telephone calls, especially long distance

- Appendix A

- Sunset & Sunrise Times at Sproul Observatory

| DATE | SUNSET (E.S.T. or E.D.T.*) | S.T. 45m after sunset | S.T. MIDNIGHT | S.T. 45m before sunset | SUNRISE (E.S.T. or E.D.T.*) | DATE |
| --- | --- | --- | --- | --- | --- | --- |
| JAN 00 | 4:44 pm | 00:11 | 06:42 | 13:19 | 7:22 am | JAN 00 |
| 10 | 4:53 | 00:59 | 07:21 | 13:58 | 7:22 | 10 |
| 20 | 5:04 | 01:49 | 08:00 | 14:33 | 7:18 | 20 |
| FEB 00 | 5:17 | 02:42 | 08:40 | 15:05 | 7:10 | FEB 00 |
| 10 | 5:29 | 03:37 | 09:23 | 15:38 | 7:00 | 10 |
| 20 | 5:41 | 04:28 | 10:03 | 16:06 | 6:48 | 20 |
| MAR 00 | 5:51 | 05:10 | 10:34 | 16:24 | 6:35 | MAR 00 |
| 10 | 6:01 | 05:59 | 11:13 | 16:49 | 6:21 | 10 |
| 20 | 6:11 | 06:49 | 11:53 | 17:13 | 6:05 | 20 |
| APR 00 | 6:21 | 07:42 | 12:36 | 17:38 | 5:47 | APR 00 |
| 10 | 7:32* | 08:33 | 13:16 | 18:02 | 6:31* | 10 |
| 20 | 7:42* | 09:22 | 13:55 | 18:26 | 6:16* | 20 |
| MAY 00 | 7:53* | 10:13 | 14:35 | 18:52 | 6:02* | MAY 00 |
| 10 | 8:03* | 11:02 | 15:14 | 19:20 | 5:51* | 10 |
| 20 | 8:12* | 11:50 | 15:53 | 19:49 | 5:41* | 20 |
| JUN 00 | 8:21* | 12:43 | 16:37 | 20:26 | 5:34* | JUN 00 |
| 10 | 8:28* | 13:29 | 17:16 | 21:02 | 5:31* | 10 |
| 20 | 8:32* | 14:13 | 17:56 | 21:42 | 5:31* | 20 |
| JUL 00 | 8:33* | 14:53 | 18:35 | 22:24 | 5:34* | JUL 00 |
| 10 | 8:30* | 15:33 | 19:14 | 23:08 | 5:39* | 10 |
| 20 | 8:25* | 16:04 | 19:54 | 23:56 | 5:47* | 20 |
| AUG 00 | 8:16* | 16:38 | 20:37 | 00:48 | 5:56* | AUG 00 |
| 10 | 8:04* | 17:06 | 21:17 | 01:38 | 6:06* | 10 |
| 20 | 7:51* | 17:32 | 21:56 | 02:26 | 6:15* | 20 |
| SEP 00 | 7:34* | 17:58 | 22:39 | 03:20 | 6:24* | SEP 00 |
| 10 | 7:18* | 18:22 | 23:19 | 04:09 | 6:33* | 10 |
| 20 | 7:02* | 18:45 | 23:58 | 04:58 | 6:45* | 20 |
| OCT 00 | 6:48* | 19:09 | 00:33 | 05:47 | 6:54* | OCT 00 |
| 10 | 6:30* | 19:32 | 01:17 | 06:36 | 7:04* | 10 |
| 20 | 6:14* | 19:56 | 01:57 | 07:27 | 7:15* | 20 |
| NOV 00 | 5:00 | 20:25 | 02:40 | 08:22 | 6:27 | NOV 00 |
| 10 | 4:49 | 20:53 | 03:19 | 09:12 | 6:38 | 10 |
| 20 | 4:41 | 21:25 | 03:59 | 10:04 | 6:50 | 20 |
| DEC 00 | 4:36 | 21:59 | 04:33 | 10:54 | 7:01 | DEC 00 |
| 10 | 4:35 | 22:38 | 05:13 | 11:43 | 7:10 | 10 |
| 20 | 4:37 | 23:19 | 05:57 | 12:29 | 7:17 | 20 |
| 30 | 4:44 | 00:11 | 06:42 | 13:19 | 7:22 | 30 |

* Times followed by asterisk occur when Daylight Savings Time is in effect between Apr 10 — Oct 20 inclusive, and are therefore given in Eastern Daylight Time (E.D.T.).    All others are given in E.S.T.

- Appendix B

- Bright Double Stars For Focusing

- Primary source: Compendium of Practical Astronomy, G.D. Roth, ed., Springer-Verlag, 1994

| ADS | RA (2000) | Dec(2000) | m | Sp | q (°) | r (¢¢) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 61 | 00h04.6m | +34°16¢ | 6.5 - 7.3 | G | 308+ | 1.5 |  |
| 683 | 49.8 | +27 43 | 6.3 - 6.3 | F | 296 | 4.4 | 65 Psc |
| 923 | 01 07.2 | - 01 44 | 7.4 - 8.2 | F | 317 | 4.2 |  |
| 1081 | 19.8 | - 00 29 | 6.4 - 7.1 | F | 12 | 1.6 | 42 Cet |
| 1438 | 49.2 | +47 54 | 6.5 - 7.0 | A | 207 | 2.0 |  |
| 1457 | 50.1 | +22 17 | 6.2 - 7.4 | F | 166 | 2.8 | 1 Ari |
| 1615 | 02 02.0 | +02 46 | 4.6 - 5.6 | A | 278 | 1.9 | a Psc |
| 1697 | 12.4 | +30 18 | 5.5 - 6.9 | G | 72 | 3.9 | i Tri |
| 2122 | 47.5 | +19 22 | 7.3 - 8.2 | G | 308 | 3.7 |  |
| 2257 | 59.2 | +21 20 | 5.2 - 5.5 | A | 207 | 1.5 | e Ari |
| 2390 | 03 12.1 | +37 12 | 8.1 - 8.3 | G | 127 | 2.6 |  |
| 2668 | 40.0 | +34 08 | 7.6 - 7.6 | F | 76 | 1.8 |  |
| 2926 | 04 00.9 | +23 13 | 6.9 - 7.8 | B | 128 | 7.3 |  |
| 2999 | 07.6 | +15 09 | 6.0 - 8.8 | A | 222 | 3.8 |  |
| 3297 | 34.6 | +18 01 | 7.0 - 7.1 | B | 277 | 3.1 |  |
| 3353 | 35.8 | +26 56 | 7.2 - 7.2 | F | 194 | 4.0 |  |
| 3734 | 05 10.4 | +37 18 | 6.9 - 7.0 | B | 221 | 1.6 |  |
| 3764 | 11.7 | +01 02 | 6.3 - 7.8 | F | 182 | 1.7 |  |
| 3991 | 23.9 | - 00 53 | 6.7 - 7.0 | F | 162 | 2.5 |  |
| 4068 | 29.2 | +25 09 | 5.9 - 6.7 | A | 206 | 4.8 | 118 Tau |
| 4123 | 31.2 | +03 17 | 5.8 - 7.1 | B | 27 | 2.0 | 33 Ori |
| 4208 | 37.2 | +26 56 | 6.4 - 6.5 | B | 327 | 1.1 |  |
| 4390 | 48.0 | +06 27 | 6.0 - 6.0 | A | 215 | 1.3 | 52 Ori |
| 4991 | 06 22.8 | +17 34 | 7.1 - 8.1 | A | 19 | 2.3 |  |
| 5436 | 48.3 | +55 43 | 6.3 - 6.3 | F | 257 | 4.7 |  |
| 5746 | 07 05.6 | +52 45 | 6.9 - 7.0 | A | 149 | 4.1 |  |
| 6126 | 29.4 | - 14 59 | 6.2 - 7.8 | F | 20 | 2.1 |  |
| 6569 | 08 05.6 | +27 32 | 6.5 - 7.4 | B | 351 | 3.5 |  |
| 6623 | 09.5 | +32 14 | 7.1 - 8.0 | F | 26 | 2.7 |  |
| 7034 | 50.9 | +35 04 | 7.5 - 7.5 | F | 278 | 3.6 |  |
| 7071 | 54.2 | +30 34 | 6.2 - 6.5 | K | 313 | 1.6 | 57 Cnc |
| 7286 | 09 18.5 | +35 22 | 6.4 - 6.7 | A | 48 | 1.8 |  |
| 7307 | 21.0 | +38 12 | 6.5 - 6.7 | F | 270+ | 1.0 |  |
| 7348 | 24.9 | +51 34 | 6.6 - 8.3 | G | 335 | 1.5 |  |
| 7380 | 27.2 | +06 14 | 7.5 - 7.5 | F | 347 | 2.2 |  |
| 7704 | 10 16.3 | +17 44 | 7.2 - 7.4 | F | 182 | 1.4 |  |
| 7837 | 35.1 | +08 39 | 5.8 - 8.5 | A | 157 | 2.2 | 49 Leo |
| 7936 | 49.3 | - 04 01 | 6.9 - 7.7 | A | 10 | 2.3 | 40 Sex |

- Appendix B

- Bright Double Stars For Focusing (continued)

| ADS | RA (2000) | Dec(2000) | m | Sp | q (°) | r (¢¢) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8119 | 11 h18.2m | +31 32 | 4.4 - 4.9 | G | 58- | 1.3 | x UMa |
| 8220 | 34.7 | +16 48 | 6.0 - 7.3 | B | 209 | 3.4 | 90 Leo |
| 8406 | 12 04.3 | +21 28 | 6.0- 7.5 | F | 237 | 3.7 | 2 Com |
| 8539 | 24.4 | +25 35 | 6.7 - 7.8 | A | 325 | 1.6 |  |
| 8695 | 53.5 | +21 15 | 5.1 - 7.3 | G | 173+ | 1.1 | 35 Com |
| 8710 | 56.3 | +54 05 | 6.0 - 7.9 | A | 283 | 3.5 |  |
| 8949 | 13 34.3 | - 00°19¢ | 7.8 - 8.7 | K | 119 | 2.2 |  |
| 9031 | 49.1 | +26 59 | 7.6 - 8.0 | K | 166+ | 3.4 |  |
| 9053 | 55.0 | - 08 04 | 6.5 - 7.7 | F | 95 | 3.4 |  |
| 9192 | 14 16.6 | +20 07 | 6.6 - 8.3 | F | 162 | 4.4 |  |
| 9396 | 49.3 | - 14 09 | 5.8 - 6.7 | A | 355 | 1.8 | m Lib |
| 9406 | 49.6 | +48 43 | 6.1 - 6.8 | F | 45 | 3.0 | 39 Boo |
| 9425 | 53.4 | +15 43 | 6.9 - 7.5 | F | 169 | 1.3 |  |
| 9507 | 15 07.5 | +09 14 | 7.4 - 7.4 | G | 211 | 4.3 |  |
| 9626 | 24.5 | +37 21 | 7.0 - 7.6 | G | 11- | 2.1 | m Boo |
| 9969 | 16 13.3 | +13 33 | 7.5 - 7.7 | K | 347 | 4.2 | 49 Ser |
| 10129 | 36.3 | +52 55 | 5.6 - 6.6 | A | 108 | 3.3 | 17 Dra |
| 10312 | 17 01.9 | +08 27 | 6.5 - 7.7 | A | 187 | 1.3 |  |
| 10597 | 29.0 | +50 53 | 7.6 - 7.8 | F | 261 | 3.1 |  |
| 10875 | 53.3 | +40 00 | 5.2 - 8.5 | K | 115 | 1.7 | 90 Her |
| 10905 | 56.4 | +18 20 | 7.3 - 7.3 | A | 293 | 2.6 |  |
| 11123 | 18 10.1 | +16 29 | 6.5 -7.5 | F | 224 | 1.2 |  |
| 11483 | 35.9 | +16 58 | 6.7 - 7.1 | G | 160 | 1.7 |  |
| 11635 | 44.3 | +39 40 | 5.1 - 6.0 | A | 353 | 2.7 | e Lyr |
| 11640 | 45.5 | +05 30 | 6.3 - 6.7 | A | 119 | 2.4 |  |
| 12169 | 19 12.1 | +49 51 | 6.6 - 6.8 | G | 213 | 8.6 |  |
| 12594 | 33.3 | +20 25 | 7.2 - 8.7 | A | 147 | 5.1 |  |
| 12962 | 48.7 | +11 49 | 6.1 - 6.9 | F | 109 | 1.3 | p Aql |
| 13256 | 20 01.4 | +10 45 | 7.5 - 7.7 | F | 352 | 4.0 |  |
| 13312 | 03.5 | +36 01 | 7.2 - 8.0 | O | 174 | 2.0 |  |
| 13672 | 18.1 | +40 44 | 6.0 - 8.2 | B | 245 | 2.8 |  |
| 14158 | 41.0 | +32 18 | 5.9 - 8.0 | K | 47 | 2.7 | 49 Cyg |
| 14421 | 54.8 | +32 42 | 8.1 - 8.2 | G | 286 | 1.1 |  |
| 14556 | 21 02.2 | +07 11 | 7.4 - 7.4 | F | 216 | 2.8 | 2 Eql |
| 14573 | 03.0 | +01 32 | 6.7 - 7.2 | F | 122 | 1.5 |  |
| 14682 | 08.6 | +30 12 | 5.7 - 7.7 | A | 306 | 3.5 |  |
| 15007 | 28.9 | +11 05 | 7.4 - 7.4 | F | 89 | 1.6 |  |
| 15767 | 22 14.5 | +07 59 | 6.8 - 8.3 | A | 122 | 1.5 |  |
| 16228 | 44.1 | +39 28 | 6.3 - 8.5 | K | 278 | 2.9 |  |
| 17149 | 23 59.5 | +33 43 | 6.5 - 6.7 | G | 322+ | 1.7 |  |

- Appendix C

- The 24-inch Refracting Telescope

- BRIEF HISTORY

- 1910    construction of    refractor begins, from donation by State Senator Sproul

- 1912    continuous operation of Sproul refractor begins in January

- 1966-7 refractor overhauled for 7 months, tracking & setting motors installed, new electronic console

- 1998    Apogee AP-6 CCD camera attached to refractor, cables run to observing alcove at dome entrance

- SPECIFICATIONS

- aperture 		0.61 m

- focal length 	11 m

- focal ratio 		f/18

- favorable for precise measurement of positions & motions of stars

- has advantage that instrument not much affected by background campus & suburban lighting

- SEEING LIMITATIONS

- optimal seeing disk on photographic plates 	~ 1.0¢¢    (typical ~ 1.5¢¢)

- optimal visual resolution 				~ 0.1¢¢    (typical ~ 0.3¢¢)

- LIMITING MAGNITUDES

- practical limiting visual magnitude 	            	~ 13

- practical limiting photographic magnitude 	~ 14

- sky background magnitude 		    	~ 15

- PAST RESEARCH PROJECTS

The Sproul Observatory has in previous years employed:

- photographic observations to determine distances of stars, especially nearby faint dwarfs

- micrometer observations of visual binary stars, their orbits & masses

- Sproul plate archive contains probably largest astrometric plate collection from one telescope:

- 90,000 photographic plates since early 1900s:

- used to make 1400 parallax (distance) determinations

- Visual observations, mainly by W.D. Heintz since 1967:

- 500 binary star orbits

- 900 newly discovered binaries

- study of low-mass binaries, including first “brown dwarf” candidates (Wolf 424 in 1972)

- Photographic program terminated in 1994

- Micrometer observations terminated in 1998

- Apogee AP-6 CCD camera installed to investigate variable stars in March 1998

- Appendix D

- The Apogee AP-6 Camera

The Apogee AP-6 Camera contains a very large chip yields workable field of view with f/18 refractor.    It was reviewed in CCD Astronomy, Summer 1996 (then Axiom AX-6).    Specifications are provided below.

- KODAK KAF 1000 CHIP

- 1024 x 1024 pixel array, 24.6 x 24.6 mm square

- 24 m square pixels

- front-illuminated, no antiblooming

- linear well depth    170Ke

- dark count ~ 1.0 e/sec at -25°C (doubles for every ~6°C warmer)

- pixel binning 1x1 to 8x8

- SYSTEM CONVERSION & MISCELLANEOUS INFORMATION

- download > 45K pixels/sec, total frame 24 sec

- readout noise < 24 e (19e typical)

- system gain 3.5 e/ADU nominal

- exposure time 0.03 sec to 10,400 sec

- frame size full frame, subframe, focus mode

- COOLING

- thermoelectric cooler with forced air.

- minimum 35°C below ambient at 25°C (remote boost unit adds 5-8°C cooling)

- 20 min temperature transition time for sensor safety

- Software programmable temperature

- CAMERA HEAD

- 8061-T6 aluminum body, hard blue anodized

- BBAR coated silica windows

- camera head is 6x6 in x 1.3 in thick (heat sinks & fans add 1.2 in)

- 3 lbs

- AP-6 CAMERA MOUNTED ON 24-INCH

- yields 8¢ x 8¢ field of view with 24-inch at f/18

- pixel scale 0.45¢¢ per pixel at focal plane

- 16-bit images, each takes up ~ 2 Mb of space

- FILTER WHEEL

- constructed by ISI Systems, Santa Barbara, CA

- contains eight, 2-inch square slots

- standard B, V, R filters, 5 mm thick,    from Omega Optical,

- PMIS SOFTWARE

- full version

- COMPUTER

- PowerSpec PC

- 233 MHz Pentium II processor

- 64 Mb RAM